Supplemental Data

Effects of compositions, chemical structures, and nanoporosity of sedimentary organic matter on degradation of benzo(a)pyrene by hydrogen peroxide

Chenya Zhuoa,b, Dainan Zhanga, Yu Yanga, Xiangyun Zhanga, Yong Rana*, Jingdong Maoc

a State Key Laboratory of Organic Geochemistry, Guangzhou Institute of Geochemistry, Chinese Academy of Sciences, Guangzhou 510640, China

b University of Chinese Academy of Sciences, Beijing 100049, China

c Department of Chemistry and Biochemistry, Old Dominion University, Norfolk, Virginia, 23529, USA

* Corresponding authors: 86-20-85290263 (phone), 86-20-85290706 (fax), e-mail: yran@gig.ac.cn (Yong Ran)

Contents:

1. Sediment collection

2. Sediment spiking with BaP and aging.

3. Determination of radioactivity in degradation experiments.

3. Figure S1. The sampling locations for the sediments.

4. Figure S2. Flow diagram of degradation experiment

5. Figure S3. Flow diagram for the separation of organic matter with different stability

6. Figure S4. Correlations of aliphatic C and aromatic C from CP spectra with aliphatic C and aromatic C from DP spectra. The data were cited from Huang et al.1

7.Figure S5. Carbon dioxide adsorption isotherms fitted to the Dubinin-Radushkevitch equation on the OS and ROC of different sediments.

8. Figure S6. Porosity distributions calculated by the CO2-DFT Model on the OS and ROC of different sediments.

9. Figure S7. The histogram of the contents of ferrous ions in sediments (a), XRD diffraction diagram in different sediments (b).

10. Figure S8. Correlations among the contents of the chemically resistant OC (ROC) and the Vo-bulk and SSA-bulk values (a-b), where Vo-bulk or SSA-bulk means nanopore volumes or specific surface areas from the ROC fractions in the bulk samples; Correlations between the Faliph-C and alky-C with the Vo-bulk and SSA-bulk values (c-f).

11.Figure S9. Residual amounts of parent compounds (BaP) in extractable residues (a) before oxidizing, and water-soluble residues (b), extractable residues (c) after oxidizing were determined by HPLC, respectively. Red: A1 sample; black: A4 sample; blue: E2 sample; gray: E3 sample; green: E4 sample; yellow: E5 sample.

12. Figure S10. Correlations of the Faliph-bulk (%), alkyl C-bulk (%), (CH2)n-bulk (%), Farom-bulk (%), arom(C-C)–bulk (%) and Arom(C-O)–bulk (%) with the degradation efficiency of 14C-BaP (%)

12. Figure S11. The relationship of the degradation kinetics parameters (Frap, Fslow) with Vo-bulk (μL/g) and SSA-bulk (m2/g).

13. Table S1. OC contents of USOC, STOC, MOC, and ROC, and their percentages.

14. Table S2. Specific surface areas (SSA), nanopores (Vo), and pore size distributions of the sediments before and after oxidation

15. Table S3. The compositions and contents of the minerals in different sediments

16. Table S4. Residual radioactivity (%) in each residue after treatment

17. Table S5. Residual amount of parent compound (BaP) in water-soluble residues and extractable residues after treatment

16. Table S6. Correlations among the SOC fractions (SOC, MOC, and ROC), functional groups of the ROC fractions, nanoporosity and degradation kinetics parameters in the bulk sediments.

1. Sediment collection. The sampling sites are presented in Figure S1. One sediment sample (X1: 23.77°N, 114.57°E) was collected in January 2012 in the Xinfengjiang reservoir. Two sediment samples (A1: 13o52.909"E, 22o 25.220"N, A4: 113o49.53" E, 22o 00.408"N) were collected in January 2012 in the Pearl River Estuary. Four sediment samplers (E2: 113°59.946"E, 21°0.035"N, E3: 114°13.563"E, 20°38.333"N, E4: 114°35.915"E, 20°0.021"N, E5: 114°53.798"E, 19°30.000"N) were collected in May in 2012 in the northern South China Sea. The water depth was 28.7 m for X1, 21 m for A1, 11m for A4, 79 m for E2, 83 m for E3, 208 m for E4, and 1468 m for E5.

2.Sediment spiking with BaP and aging. The spiked process is briefly described as follows: 20 g (dw, dry weight) of a given sediment was weighted to a 250 ml conical flask, spiked by 50 ng/g of 14C-BaP and 450 ng/g of non-radioactive labeled BaP stock solution in acetone, and artificially stirred for 1 h until it was fully mixed. This mixture was placed in a fume hood overnight so that acetone solvent was allowed to be evaporated. A given amount of deionized water was added, aged for one month. Afterwards, the samples were freeze-dried and ground through an 80-mesh sieve for the degradation experiment. In order to examine the homogeneity of BaP in the sediments, we randomly selected 0.05 g of each sample, and burned at 900 °C on a Model A307 Sample Oxidizer (Perkin Elmer, USA) for 5 min. The generated 14CO2 was absorbed with 15 mL of alkaline cocktail (Perkin Elmer, USA), and the radioactivity was counted on a liquid scintillation counter (LSC) (Perkin Elmer, USA). The relative standard deviation of radioactivity for sediment replicates ranges from 1.40% to 3.65%, indicating that BaP was homogeneously distributed in the spiked samples.

3. Determination of radioactivity in degradation experiments. Each of the extracts was shaken and concentrated to 2-3 mL by evaporation, then mixed with 10 ml of scintillation cocktail (Perkin Elmer, USA) and counted on liquid scintillation counter (LSC) (Perkin Elmer, USA). For radioactivity in aqueous samples, 5 mL of water samples and 1 mL of alkaline solution (14CO2) were mixed with 10mL and 3mL cocktail, respectively. For radioactivity in bound residues, 0.5 g of extracted sediment was combusted using a Model A307 Sample Oxidizer (Perkin Elmer, USA) for 5 min and the generated 14CO2 was absorbed with 15 mL of alkaline cocktail (Perkin Elmer, USA), and counted on LSC.

Figure S1. The sampling locations for the sediments

Figure S2. Flow diagram of degradation experiment

Figure S3. Flow diagram for the separation of organic matter fractions.

Figure S4. Correlations of aliphatic C and aromatic C from CP spectra with aliphatic C and aromatic C from DP spectra. The data were cited from Huang et al.1

Figure S5. Carbon dioxide adsorption isotherms fitted to the Dubinin-Radushkevitch equation on the OS and ROC of different sediments.

Figure S6. Porosity distribution calculated by CO2-DFT Model on the OS and ROC of different sediments.

Figure S7. Correlations among the contents of the ROC and the Vo-bulk and SSA-bulk values (a-b), where Vo-bulk or SSA-bulk means nanopore volumes or specific surface areas from the ROC fractions in the bulk samples; Correlations between the Faliph-C and alky-C with the Vo-bulk and SSA-bulk values (c-f).

Figure S8. The histogram of the contents of ferrous ions in sediments (a), XRD diffraction diagram in different sediments (b).

Figure S9. Residual amounts of parent compounds (BaP) in extractable residues (a) before oxidation, and water-soluble residues (b), extractable residues (c) after oxidation, which were determined by HPLC, respectively. Red: A1 sample; black: A4 sample; blue: E2 sample; gray: E3 sample; green: E4 sample; yellow: E5 sample.

Figure S10. Correlations of the Faliph-bulk (%), alkyl C-bulk (%), (CH2)n-bulk (%), Farom-bulk (%), arom(C-C)–bulk (%) and arom(C-O)–bulk (%) with the degradation efficiency of 14C-BaP (%).

Figure S11. The relationships of the degradation kinetics parameters (Frap, Fslow) with Vo-bulk (μL/g) and SSA-bulk (m2/g).

| Table S1 OC contents of USOC, STOC, MOC, and ROC) in the bulk, and the percentages accounting for the bulk OC in the six sediment samples |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OC(Bulk) | a USOC | USOC/OC | STOC | STOC/OC | bMOC | MOC/OC | ROC | ROC/OC |
|  | c g.kg-1 | g.kg-1 | % | g.kg-1 | % | g.kg-1 | % | g.kg-1 | % |
| A01 | 8.91 | 6.75 | 75.80 | 2.16 | 24.20 | 1.37 | 15.38 | 0.78 | 8.80 |
| A04 | 8.58 | 4.59 | 53.50 | 3.99 | 46.50 | 2.99 | 34.80 | 1.09 | 12.70 |
| E2 | 2.55 | 1.24 | 48.60 | 1.31 | 51.40 | 0.11 | 4.30 | 1.20 | 47.10 |
| E3 | 3.14 | 1.08 | 34.40 | 2.06 | 65.6 | 0.42 | 13.40 | 1.64 | 52.20 |
| E4 | 1.71 | 0.62 | 36.30 | 1.09 | 63.70 | 0.11 | 6.43 | 0.98 | 57.30 |
| E5 | 6.89 | 0.38 | 5.52 | 6.51 | 94.50 | 1.52 | 22.10 | 4.99 | 72.40 |
| a: USOC = Bulk – SOC、 b: MOC = SOC –ROC、c:g-OC/kg-bulk-sediment |  |  |  |  |  |  |  |  |  |

| Table S2. specific surface area (SSA), nanopore (Vo), pore size distribution, and calibration volume (Vo (CO2-N2)) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| for the OS and the ROC fractions |  |  |  |  |  |  |  |  |  |
| sample | Dubinin-radushkevitch (DR) |  |  |  |  | cumulative volume (CV) |  |  |  |
|  | Vo | SSA | CSA | Vo (CO2-N2) |  | 0-1.1nm | 0-0.7nm | 0.7-1.1nm |  |
|  | μL/g | m2/g | m2/g | μL/g |  | μL/g | μL/g (%of 0-1.1nm) | μL/g (% of 0-1.1nm) |  |
|  |  |  |  |  | OS |  |  |  |  |  |  |
| A1 | 6.24 | 15.6 | 5.44 | 5.18 |  | 1.61 | 1.23 (76.5) | 0.38 (23.4) |  |
| A4 | 7.28 | 18.2 | 4.89 | 6.22 |  | 1.47 | 1.13 (76.9) | 0.34 (23.1) |  |
| E2 | 2.71 | 6.76 | 1.91 | 2.29 |  | 0.56 | 0.46 (82.1) | 0.10 (17.9) |  |
| E3 | 3.32 | 8.29 | 0.97 | 3.14 |  | 0.28 | 0.24 (86.0) | 0.04 (14.0) |  |
| E4 | 5.25 | 13.1 | 3.46 | 3.62 |  | 0.99 | 0.80 (80.8) | 0.19 (19.2) |  |
| E5 | 10.77 | 26.9 | 12.42 | 8.81 |  | 3.51 | 2.76 (78.7) | 0.75 (21.3) |  |
|  |  |  |  |  | ROC |  |  |  |  |  |  |
| A1 | 7.13 | 19.3 | 10.40 | 5.30 |  | 3.07 | 2.36 (76.9) | 0.71 (23.1) |  |
| A4 | 8.01 | 20.0 | 9.31 | 6.51 |  | 2.76 | 2.14 (77.5) | 0.62 (22.5) |  |
| E2 | 11.1 | 27.6 | 5.98 | 10.3 |  | 1.80 | 1.42 (78.6) | 0.39 (21.4) |  |
| E3 | 10.7 | 26.6 | 8.09 | 10.4 |  | 2.45 | 1.86 (75.8) | 0.59 (24.2) |  |
| E4 | 10.7 | 26.7 | 6.05 | 7.85 |  | 1.82 | 1.39 (76.6) | 0.43 (23.4) |  |
| E5 | 15.0 | 37.5 | 15.90 | 13.0 |  | 4.42 | 3.73 (84.3) | 0.69 (15.7) |  |

| Table S3. The compositions and contents of the mineral in different sediments |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NO. | Quartz (%) | Illite (%) | Albite (%) | Calcite (%) | pyrite | siderite |
| A1 | 89.8 | 10.2 | N.D. | N.D. | N.D. | N.D. |
| A4 | 76.3 | 10.9 | 8.3 | 4.5 | N.D. | N.D. |
| E2 | 69.4 | 10.2 | 14.7 | 5.7 | N.D. | N.D. |
| E3 | 26.5 | 13.4 | 26.6 | 9.8 | N.D. | 23.8 |
| E4 | 36.9 | 14.7 | 17.4 | 13.8 | 0.5 | 11.9 |
| E5 | 35.3 | 16.7 | 22.5 | 25.5 | N.D. | N.D. |

| Table S4. Residual radioactivity (%) in each residue after the treatment |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| % Recovery | X1 | A1 | A4 | E2 | E3 | E4 | E5 |
| Degradation to 14CO2 | 81.00 | 60.80 | 23.80 | 15.80 | 19.90 | 11.30 | 1.42 |
| Extractable residues | 5.40 | 13.81 | 30.66 | 41.78 | 37.57 | 40.99 | 60.56 |
| Water-soluble residues | 5.30 | 6.02 | 20.32 | 16.16 | 21.60 | 19.90 | 4.19 |
| Bound residues | 4.30 | 17.81 | 19.37 | 23.74 | 17.93 | 25.23 | 30.58 |
| Total recovery | 96.00 | 98.44 | 94.15 | 97.48 | 97.00 | 97.43 | 96.75 |
| The radioactivity in extractable residues, water-soluble residues and bound residues may include intermediates and parent compounds. |  |  |  |  |  |  |  |

| Table S5. Residual amount of parent compound (BaP) in water-soluble residues and |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| extractable residues after treatment |  |  |  |  |  |  |  |
| % Recovery | X1 | A1 | A4 | E2 | E3 | E4 | E5 |
| Extractable residues | 4.10 | 11.51 | 12.17 | 26.50 | 22.34 | 36.17 | 57.56 |
| Water-soluble residues | nd | nd | nd | 1.11 | nd | nd | 0.12 |

| Table S6. Correlations among the SOM fractions (SOC, MOC, and ROC), functional groups of the ROC fractions, nanoporosity, and degradation kinetics parameters in the bulk sediments. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STOCa | MOCa | ROCa | Farom-bulkb | Faliph-bulkb | alkyl C-bulkb | arom(C-C)-bulkb | Vo-bulka | SSA-bulka | Frapa | krapa | Fslowa | kslowa |
| STOCa | 1 | 0.435 | 0.926** | -0.791* | 0.894** | 0.958** | -0.859* | 0.882** | 0.912** | -0.762* | -0.627 | 0.765* | -0.678 |
| MOCa |  | 1 | 0.200 | -0.399 | 0.398 | 0.583 | -0.339 | 0.617 | 0.629 | -0.547 | -0.235 | 0.559 | -0.371 |
| ROCa |  |  | 1 | -0.848* | 0.840* | 0.825* | -0.831* | 0.714* | 0.746* | -0.651 | -0.580 | 0.651 | -0.599 |
| Farom-bulkb |  |  |  | 1 | -0.975** | -0.841* | 0.959** | -0.801* | -0.859* | 0.943** | 0.836* | -0.937** | 0.499 |
| Faliph-bulkb |  |  |  |  | 1 | 0.917** | -0.969** | 0.915** | 0.879** | -0.884** | -0.713 | 0.884** | -0.467 |
| alkyl C-bulkb |  |  |  |  |  | 1 | -0.835* | 0.991** | 0.995** | -0.797* | -0.513 | 0.807* | -0.398 |
| arom(C-C)-bulkb |  |  |  |  |  |  | 1 | -0.819* | -0.780* | 0.880** | 0.694 | -0.882** | 0.608 |
| Vo-bulka |  |  |  |  |  |  |  | 1 | 0.991** | -0.783* | -0.572 | 0.784* | -0.373 |
| SSA-bulka |  |  |  |  |  |  |  |  | 1 | -0.714* | -0.414 | 0.718* | -0.358 |
| Frapa |  |  |  |  |  |  |  |  |  | 1 | 0.511 | -0.999** | 0.509 |
| Krapa |  |  |  |  |  |  |  |  |  |  | 1 | -0.483 | 0.271 |
| Fslowa |  |  |  |  |  |  |  |  |  |  |  | 1 | 0.582 |
| kslowa |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| **Correlation is significant at the 0.01 level. * Correlation is significant at the 0.05 level. a: n = 6, b: n = 5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Farom-bulk, Faliph-bulk, alkyl C-bulk and Arom(C-C)-bulk refer to the the contents of aliphatic C, aromatic C, alky C and arom(C-C) from the NHC fractions in the bulk respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference

1.	Huang, Y.; Zhang, D.; Duan, D.; Yu, Y.; Xiong, Y.; Yong, R., Importance of the structure and nanoporosity of organic matter on the desorption kinetics of benzo[ a ]pyrene in sediments Environ. Pollut. 2017, 225, 628.