Table 8.2 CT Numbers of Core#8 for Saturation Calculations.

| Slice | CT number of 100 % Water Saturated Core | CT number of 100 % n-decane Saturated Core | CT number of core with irreducible water saturation | Water saturation initially (percent) | CT number of waterflooded Core | Water saturation after waterflood (percent) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 610 | 600 |  | 615 |  |
| 2 | 1,386 | 1,100 | 1,285 | 65 | 1,360 | 91 |
| 3 | 1,400 | 1,130 | 1,283 | 57 | 1,372 | 90 |
| 4 | 1,386 | 1,119 | 1,238 | 45 | 1,317 | 74 |
| 5 | 1,388 | 1,115 | 1,242 | 47 | 1,306 | 70 |
| 6 | 1,384 | 1,112 | 1,233 | 44 | 1,292 | 66 |
| 7 | 1,381 | 1,115 | 1,218 | 39 | 1,277 | 61 |
| 8 | 1,387 | 1,116 | 1,237 | 45 | 1,285 | 63 |
| 9 | 1,396 | 1,129 | 1,264 | 50 | 1,309 | 68 |
| 10 | 1,399 | 1,128 | 1,265 | 51 | 1,306 | 66 |
| 11 | 1,388 | 1,124 | 1,239 | 43 | 1,278 | 58 |
| 12 | 1,385 | 1,120 | 1,244 | 47 | 1,274 | 58 |
| 13 | 1,380 | 1,107 | 1,228 | 44 | 1,257 | 55 |
| 14 | 1,384 | 1,120 | 1240 | 45 | 1,273 | 58 |
| 15 | 1,386 | 1,117 | 1,247 | 48 | 1,275 | 59 |
| 16 | 610 | 611 | 605 |  | 605 |  |
| average water saturation (percent) |  |  |  | 47.82 |  | 66.83 |

Table 8.3 CT Numbers of Core#2 for Saturation Calculations.

| Slice | CT number of 100 % Water Saturated Core | CT number of 100 % n-decane Saturated Core | CT number of core with irreducible water saturation | Water saturation initially (percent) | CT number of waterflooded Core | Water saturation after waterflood (percent) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 620 | -285 | 590 |  | 615 |  |
| 2 | 1,400 | 1,100 | 1,253 | 61 | 1,300 | 80 |
| 3 | 1,387 | 1,071 | 1,234 | 48 | 1,276 | 68 |
| 4 | 1,370 | 1,067 | 1,221 | 46 | 1,253 | 60 |
| 5 | 1,374 | 1,066 | 1,234 | 51 | 1,267 | 65 |
| 6 | 1,366 | 1,073 | 1,240 | 53 | 1,272 | 66 |
| 7 | 1,366 | 1,075 | 1,253 | 56 | 1,282 | 68 |
| 8 | 1,361 | 1,068 | 1,241 | 54 | 1,269 | 66 |
| 9 | 1,370 | 1,054 | 1,228 | 45 | 1,262 | 61 |
| 10 | 1,376 | 1,065 | 1,241 | 51 | 1,263 | 61 |
| 11 | 1,366 | 1,071 | 1,226 | 47 | 1,250 | 58 |
| 12 | 1,371 | 1,071 | 1,250 | 55 | 1,275 | 66 |
| 13 | 1,368 | 1,071 | 1,265 | 62 | 1,282 | 68 |
| 14 | 1,367 | 1,073 | 1,268 | 61 | 1,298 | 73 |
| 15 | 1,375 | 1,058 | 1,215 | 46 | 1,250 | 63 |
| 16 | 615 | -285 | 605 |  | 605 |  |
| average water saturation (percent) |  |  |  | 52.63 |  | 65.88 |

Table 8.4 CT Numbers of Core#2 & #8 for Saturation Calculations.

|  | Water saturation (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Application | n-decane injection |  |  | Waterflooding |  |  |
|  | Initial % | Final % | Difference % | Initial % | Final % | Difference % |
| Core #2 | 100 | 52.63 | 47.37 | 52.63 | 65.88 | 13.25 |
| Core #8 | 100 | 47.82 | 52.18 | 47.82 | 66.83 | 19.01 |

Figure 8.2 CT Scan Images of Core#8, Dry (CTdry).

Figure 8.3 CT Scan Images of  Core#8, 100% Water Saturated (CTwsat).

Figure 8.4 CT Scan Images of  Core#8, 100% n-Decane Saturated (CTo).

Figure 8.5 CT Scan Images of  Core#8, Irreducible Water Saturated (CTx).

Figure 8.6 CT Scan Images of  Core#8, Waterflooded (CTx).

Figure 8.18 CT Scan Images Arificially Fractured Core#3 Left Half Dry (CTdry).

Figure 8.19 CT Scan Images of  Arificially Fractured Core#3 Right Half Dry (CTdry).

Figure 8.20 CT Scan Images of Arificially Fractured Core#3 Left Half Water Saturated(CTwsat).

Figure 8.21 CT Scan Images of Arificially Fractured Core#3 Right Half Water Saturated (CTwsat).

Table 8.9 CT Numbers of Artificially Fractured Core#7

Left Matrix, Dry (CTdry)and Water Saturated ( CTwsat).

| slice | Dry Scanned | Water Saturated | Porosity |
| --- | --- | --- | --- |
| # | CT# | CT# | fraction |
| 1 | -11 | -10 |  |
| 2 | 1,158 | 1,453 | 0.296 |
| 3 | 1,152 | 1,503 | 0.351 |
| 4 | 1,148 | 1,493 | 0.345 |
| 5 | 1,148 | 1,491 | 0.343 |
| 6 | 1,162 | 1,483 | 0.32 |
| 7 | 1,159 | 1,475 | 0.316 |
| 8 | 1,154 | 1,460 | 0.306 |
| 9 | 1,156 | 1,468 | 0.312 |
| 10 | 1,148 | 1,481 | 0.332 |
| 11 | 1,145 | 1,427 | 0.282 |
| 12 | 1,153 | 1,512 | 0.359 |
| 13 | 1,154 | 1,505 | 0.351 |
| 14 | 1,150 | 1,525 | 0.375 |
| 15 | 1,145 | 1,350 | 0.205 |
| 16 | -1 | 0 |  |
|  |  | average porosity | 32.09% |

# APPENDIX-A

## CORE SAMPLE XRF ANALYSIS

Preset Sample Data

Sample Name	scanbolat	Dilution Material	M-HWC

Description		Sample Mass (g)	6.2500

Method press	Dilution Mass (g)	1.4000

Job Number	0	Dilution Factor	0.8170

Sample State	Pressed tablet, 40 mm	Sample rotation	Yes

Sample Type	Pellet_40	Date of Receipt	09/22/2016

Sample Status	AAXXXX	Date of Evaluation	09/22/2016

Table A.1 Core Sample Element Analysis.

| Z | Symbol | Element | Concentration % | Abs. Error % |
| --- | --- | --- | --- | --- |
| 11 | Na2O | Sodiumoxide | 7.57 | 0.14 |
| 12 | MgO | Magnesiumoxide | 1.220 | 0.028 |
| 13 | Al2O3 | Aluminumoxide | 16.68 | 0.02 |
| 14 | SiO2 | Siliconoxide | 58.65 | 0.04 |
| 15 | P2O5 | Phosphorusoxide | 0.0657 | 0.0037 |
| 16 | SO3 | Sulfurtrioxide | 0.1420 | 0.0009 |
| 17 | Cl | Chlorine | 0.1484 | 0.0004 |
| 19 | K2O | Potassiumoxide | 4.4240 | 0.015 |
| 20 | CaO | Calciumoxide | 6.032 | 0.013 |
| 22 | TiO2 | Titaniumdioxide | 0.7547 | 0.0034 |
| 23 | V2O5 | Vanadiumoxide | 0.00740 | 0.00100 |
| 24 | Cr2O3 | Chromiumoxide | 0.1166 | 0.0035 |
| 25 | MnO | Manganeseoxide | 0.0781 | 0.0027 |
| 26 | Fe2O3 | Ironoxide | 3.5330 | 0.007 |
| 27 | CoO | Cobaltoxide | 0.0862 | 0.0076 |
| 28 | NiO | Nickeloxide | 0.03753 | 0.00093 |
| 29 | CuO | Copperoxide | 0.0184 | 0.0013 |
| 30 | ZnO | Zincoxide | 0.0197 | 0.0014 |
| 31 | Ga | Gallium | < 0.00010 | (0.0) |
| 32 | Ge | Germanium | < 0.00010 | (0.0) |
| 33 | As2O3 | Arsenicoxide | 0.00668 | 0.00031 |
| 34 | Se | Selenium | 0.00016 | 0.00010 |
| 35 | Br | Bromine | 0.00101 | 0.00014 |
| 37 | Rb2O | Rubidiumoxide | 0.01096 | 0.00018 |
| 38 | SrO | Strontiumoxide | 0.04818 | 0.00029 |
| 39 | Y | Yttrium | 0.00383 | 0.00034 |

Table A.1 continued Core Sample Element Analysis.

| Z | Symbol | Element | Concentration % | Abs. Error % |
| --- | --- | --- | --- | --- |
| 40 | ZrO2 | Zirconiumoxide | < 0.068 | (0.058) |
| 41 | Nb2O5 | Niobiumoxide | 0.00428 | 0.00015 |
| 42 | Mo | Molybdenum | < 0.0010 | (0.00050) |
| 46 | Pd | Palladium | 0.00 | 0.00 |
| 47 | Ag | Silver | 0.00144 | 0.00002 |
| 48 | Cd | Cadmium | < 0.00051 | (0.0) |
| 49 | In | Indium | < 0.00051 | (0.0) |
| 50 | SnO2 | Tinoxide | 0.00891 | 0.00027 |
| 51 | Sb2O5 | Antimonyoxide | 0.00158 | 0.00031 |
| 52 | Te | Tellurium | 0.0209 | 0.0029 |
| 53 | I | Iodine | < 0.00071 | (0.0) |
| 55 | Cs | Cesium | < 0.00081 | (0.0) |
| 56 | Ba | Barium | 0.088 | 0.088 |
| 57 | La | Lanthanum | < 0.0010 | (0.0) |
| 58 | Ce | Cerium | 0.01132 | 0.00098 |
| 72 | Hf | Hafnium | 0.0202 | 0.0014 |
| 73 | Ta2O5 | Tantalumoxide | 0.1263 | 0.0032 |
| 74 | WO3 | Tungstenoxide | < 0.00025 | (0.0) |
| 80 | Hg | Mercury | < 0.00020 | (0.0) |
| 81 | Tl | Thallium | 0.00127 | 0.00012 |
| 82 | PbO | Leadoxide | 0.00453 | 0.00028 |
| 83 | Bi | Bismuth | < 0.00020 | (0.0) |
| 90 | Th | Thorium | 0.00333 | 0.00029 |
| 92 | U | Uranium | 0.00176 | 0.00017 |
|  |  | Sum | 100.00 |  |

# APPENDIX-B

## Initially Water Saturated Decane Flooded Naturally Fractured CT Scan Images of Core#2

Figure B.1 CT Slice Views of  Core#2 Dry.

Figure B.2 CT Slice Views of  Core#2 Water Saturated.

Figure B.3 CT Slice Views of  Core#2 Decane Flooded.

## Initially Decane Saturated Water Flooded Naturally Fractured CT Scan Images of Core#2

Figure B.4 CT Slice Views of  Core#2 Dry Scanned.

Figure B.5 CT Slice Views of  Core#2 Decane Saturated.

Figure B.6 CT Slice Views of  Core#2 Water Flooded.

## Initially Water Saturated Decane Flooded Naturally Fractured CT Numbers of Core#2

Table B.1 CT Numbers of Slices for  Core#2 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21 dry | area | dry mean | CT# | 22 water | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,260 | 184,441 | -90 | 1 | 40,260 | 173,764 | 620 |  |
| 2 | 40,260 | 135,935 | 1,075 | 2 | 40,260 | 77,924 | 1,400 | 0.3250 |
| 3 | 40,260 | 138,254 | 1,057 | 3 | 40,260 | 81,484 | 1,387 | 0.3298 |
| 4 | 40,260 | 141,596 | 1,032 | 4 | 40,260 | 86,326 | 1,370 | 0.3378 |
| 5 | 40,260 | 142,269 | 1,027 | 5 | 40,260 | 85,123 | 1,374 | 0.3472 |
| 6 | 40,260 | 141,967 | 1,029 | 6 | 40,260 | 87,259 | 1,366 | 0.3372 |
| 7 | 40,260 | 143,626 | 1,017 | 7 | 40,260 | 87,481 | 1,366 | 0.3490 |
| 8 | 40,260 | 142,503 | 1,025 | 8 | 40,260 | 88,883 | 1,361 | 0.3355 |
| 9 | 40,260 | 142,034 | 1,029 | 9 | 40,260 | 86,183 | 1,370 | 0.3416 |
| 10 | 40,260 | 139,715 | 1,046 | 10 | 40,260 | 84,714 | 1,376 | 0.3293 |
| 11 | 40,260 | 141,867 | 1,030 | 11 | 40,260 | 87,241 | 1,366 | 0.3365 |
| 12 | 40,260 | 140,992 | 1,037 | 12 | 40,260 | 85,981 | 1,371 | 0.3344 |
| 13 | 40,260 | 140,957 | 1,037 | 13 | 40,260 | 86,801 | 1,368 | 0.3312 |
| 14 | 40,260 | 139,655 | 1,047 | 14 | 40,260 | 87,160 | 1,367 | 0.3200 |
| 15 | 40,260 | 137,353 | 1,064 | 15 | 40,260 | 84,894 | 1,375 | 0.3107 |
| 16 | 40,260 | 175,000 | -120 | 16 | 40,260 | 170,771 | 615 |  |
|  |  |  |  |  |  |  | average  | 33.32% |

Table B.2 CT Numbers of Slices for  Core#2 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2w1 | area | 100 % water saturated mean | CT# | 2d2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,260 | 173,764 | 620 | 1 | 40,260 | 158,25 | -285 |
| 2 | 40,260 | 77,924 | 1,400 | 2 | 40,260 | 76,938 | 1,100 |
| 3 | 40,260 | 81,484 | 1,387 | 3 | 40,260 | 85,932 | 1,071 |
| 4 | 40,260 | 86,326 | 1,370 | 4 | 40,260 | 87,334 | 1,067 |
| 5 | 40,260 | 85,123 | 1,374 | 5 | 40,260 | 87,683 | 1,066 |
| 6 | 40,260 | 87,259 | 1,366 | 6 | 40,260 | 85,447 | 1,073 |
| 7 | 40,260 | 87,481 | 1,366 | 7 | 40,260 | 84,707 | 1,075 |
| 8 | 40,260 | 88,883 | 1,361 | 8 | 40,260 | 86,819 | 1,068 |
| 9 | 40,260 | 86,183 | 1,370 | 9 | 40,260 | 91,341 | 1,054 |
| 10 | 40,260 | 84,714 | 1,376 | 10 | 40,260 | 87,925 | 1,065 |
| 11 | 40,260 | 87,241 | 1,366 | 11 | 40,260 | 85,942 | 1,071 |
| 12 | 40,260 | 85,981 | 1,371 | 12 | 40,260 | 85,957 | 1,071 |
| 13 | 40,260 | 86,801 | 1,368 | 13 | 40,260 | 85,883 | 1,071 |
| 14 | 40,260 | 87,160 | 1,367 | 14 | 40,260 | 85,503 | 1,073 |
| 15 | 40,260 | 84,894 | 1,375 | 15 | 40,260 | 89,969 | 1,058 |
| 16 | 40,260 | 170,771 | 615 | 16 | 40,260 | 171,309 | -285 |

Table B.3 CT Numbers of Slices for Core#2 Saturation Calculation.

| 2wd | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 160,746 | 590 |  |  |
| 2 | 40,260 | 83,943 | 1,253 | 61% | 39% |
| 3 | 40,260 | 85,776 | 1,234 | 48% | 52% |
| 4 | 40,260 | 87,079 | 1,221 | 46% | 54% |
| 5 | 40,260 | 85,773 | 1,234 | 51% | 49% |
| 6 | 40,260 | 85,16 | 1,240 | 53% | 47% |
| 7 | 40,260 | 83,954 | 1,253 | 56% | 44% |
| 8 | 40,260 | 85,12 | 1,241 | 54% | 46% |
| 9 | 40,260 | 86,416 | 1,228 | 45% | 55% |
| 10 | 40,260 | 85,113 | 1,241 | 51% | 49% |
| 11 | 40,260 | 86,576 | 1,226 | 47% | 53% |
| 12 | 40,260 | 84,254 | 1,250 | 55% | 45% |
| 13 | 40,260 | 82,755 | 1,265 | 62% | 38% |
| 14 | 40,260 | 82,507 | 1,268 | 61% | 39% |
| 15 | 40,260 | 87,622 | 1,215 | 46% | 54% |
| 16 | 40,260 | 170,548 | 605 |  |  |
|  |  |  |  | 52.63% | 47.37% |

Table B.4 CT Numbers of Slices for Core#2 Saturation Calculation.

| 2wdw | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 176,022 | 615 |  |  |
| 2 | 40,260 | 80,209 | 1,300 | 80% | 20% |
| 3 | 40,260 | 82,592 | 1,276 | 68% | 32% |
| 4 | 40,260 | 84,792 | 1,253 | 60% | 40% |
| 5 | 40,260 | 83,431 | 1,267 | 65% | 35% |
| 6 | 40,260 | 82,925 | 1,272 | 66% | 34% |
| 7 | 40,260 | 81,949 | 1,282 | 68% | 32% |
| 8 | 40,260 | 83,238 | 1,269 | 66% | 34% |
| 9 | 40,260 | 83,939 | 1,262 | 61% | 39% |
| 10 | 40,260 | 83,824 | 1,263 | 61% | 39% |
| 11 | 40,260 | 85,102 | 1,250 | 58% | 42% |
| 12 | 40,260 | 82,626 | 1,275 | 66% | 34% |
| 13 | 40,260 | 81,989 | 1,282 | 68% | 32% |
| 14 | 40,260 | 80,394 | 1,298 | 73% | 27% |
| 15 | 40,260 | 85,072 | 1,250 | 63% | 37% |
| 16 | 40,260 | 170,972 | 605 |  |  |
|  |  |  |  | 65.88% | 34.12% |

## Initially Decane Saturated Decane Flooded Naturally Fractured CT Numbers of Core#2

Table B.5 CT Numbers of Slices for Core#2 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21dry | area | dry mean | CT# | 22 water | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,260 | 184,441 | -90 | 1 | 40,260 | 173,764 | 620 |  |
| 2 | 40,260 | 135,935 | 1,075 | 2 | 40,260 | 77,924 | 1,400 | 0.3250 |
| 3 | 40,260 | 138,254 | 1,057 | 3 | 40,260 | 81,484 | 1,387 | 0.3298 |
| 4 | 40,260 | 141,596 | 1,032 | 4 | 40,260 | 86,326 | 1,370 | 0.3378 |
| 5 | 40,260 | 142,269 | 1,027 | 5 | 40,260 | 85,123 | 1,374 | 0.3472 |
| 6 | 40,260 | 141,967 | 1,029 | 6 | 40,260 | 87,259 | 1,366 | 0.3372 |
| 7 | 40,260 | 143,626 | 1,017 | 7 | 40,260 | 87,481 | 1,366 | 0.3490 |
| 8 | 40,260 | 142,503 | 1,025 | 8 | 40,260 | 88,883 | 1,361 | 0.3355 |
| 9 | 40,260 | 142,034 | 1,029 | 9 | 40,260 | 86,183 | 1,370 | 0.3416 |
| 10 | 40,260 | 139,715 | 1,046 | 10 | 40,260 | 84,714 | 1,376 | 0.3293 |
| 11 | 40,260 | 141,867 | 1,030 | 11 | 40,260 | 87,241 | 1,366 | 0.3365 |
| 12 | 40,260 | 140,992 | 1,037 | 12 | 40,260 | 85,981 | 1,371 | 0.3344 |
| 13 | 40,260 | 140,957 | 1,037 | 13 | 40,260 | 86,801 | 1,368 | 0.3312 |
| 14 | 40,260 | 139,655 | 1,047 | 14 | 40,260 | 87,160 | 1,367 | 0.3200 |
| 15 | 40,260 | 137,353 | 1,064 | 15 | 40,260 | 84,894 | 1,375 | 0.3107 |
| 16 | 40,260 | 175,000 | -120 | 16 | 40,260 | 170,771 | 615 |  |
|  |  |  |  |  |  |  | average | 33.32% |

Table B.6 CT Numbers of Slices for Core#2 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2w1 | area | 100 % water saturated mean | CT# | 2d2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,260 | 173,764 | 620 | 1 | 40,260 | 158,25 | -285 |
| 2 | 40,260 | 77,924 | 1,400 | 2 | 40,260 | 76,938 | 1,100 |
| 3 | 40,260 | 81,484 | 1,387 | 3 | 40,260 | 85,932 | 1,071 |
| 4 | 40,260 | 86,326 | 1,370 | 4 | 40,260 | 87,334 | 1,067 |
| 5 | 40,260 | 85,123 | 1,374 | 5 | 40,260 | 87,683 | 1,066 |
| 6 | 40,260 | 87,259 | 1,366 | 6 | 40,260 | 85,447 | 1,073 |
| 7 | 40,260 | 87,481 | 1,366 | 7 | 40,260 | 84,707 | 1,075 |
| 8 | 40,260 | 88,883 | 1,361 | 8 | 40,260 | 86,819 | 1,068 |
| 9 | 40,260 | 86,183 | 1,370 | 9 | 40,260 | 91,341 | 1,054 |
| 10 | 40,260 | 84,714 | 1,376 | 10 | 40,260 | 87,925 | 1,065 |
| 11 | 40,260 | 87,241 | 1,366 | 11 | 40,260 | 85,942 | 1,071 |
| 12 | 40,260 | 85,981 | 1,371 | 12 | 40,260 | 85,957 | 1,071 |
| 13 | 40,260 | 86,801 | 1,368 | 13 | 40,260 | 85,883 | 1,071 |
| 14 | 40,260 | 87,160 | 1,367 | 14 | 40,260 | 85,503 | 1,073 |
| 15 | 40,260 | 84,894 | 1,375 | 15 | 40,260 | 89,969 | 1,058 |
| 16 | 40260 | 170,771 | 615 | 16 | 40,260 | 171,309 | -285 |

Table B.7 CT Numbers of Slices for Core#2 Saturation Calculation (ios).

| 2dw | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 192,77 | -50 |  |  |
| 2 | 40,260 | 133,456 | 1,278 | 59% | 41% |
| 3 | 40,260 | 137,707 | 1,269 | 63% | 37% |
| 4 | 40,260 | 140,918 | 1,263 | 65% | 35% |
| 5 | 40,260 | 138,472 | 1,268 | 66% | 34% |
| 6 | 40,260 | 140,068 | 1,265 | 65% | 35% |
| 7 | 40,260 | 139,067 | 1,267 | 66% | 34% |
| 8 | 40,260 | 141,837 | 1,261 | 66% | 34% |
| 9 | 40,260 | 143,311 | 1,258 | 65% | 35% |
| 10 | 40,260 | 142,651 | 1,260 | 63% | 37% |
| 11 | 40,260 | 141,779 | 1,261 | 64% | 36% |
| 12 | 40,260 | 140,471 | 1,264 | 64% | 36% |
| 13 | 40,260 | 141,329 | 1,262 | 64% | 36% |
| 14 | 40,260 | 137,367 | 1,270 | 67% | 33% |
| 15 | 40,260 | 140,338 | 1,264 | 65% | 35% |
| 16 | 40,260 | 175,665 | -60 |  |  |
|  |  |  |  | 59.60% | 40.40% |

Initially Water Saturated Decane Flooded CT Numbers of Core#8

Table B.8 CT Numbers of Slices for  Core#8 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 81 | area | dry mean | CT# | 82 water | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,260 | 182,012 | 580 | 1 | 40,260 | 169,219 |  |  |
| 2 | 40,260 | 144,024 | 1,050 | 2 | 40,260 | 87,552 | 1,386 | 0.3355 |
| 3 | 40,260 | 143,745 | 1,052 | 3 | 40,260 | 83,939 | 1,400 | 0.3482 |
| 4 | 40,260 | 143,968 | 1,051 | 4 | 40,260 | 87,376 | 1,386 | 0.3356 |
| 5 | 40,260 | 141,963 | 1,061 | 5 | 40,260 | 86,882 | 1,388 | 0.3270 |
| 6 | 40,260 | 140,872 | 1,067 | 6 | 40,260 | 87,918 | 1,384 | 0.3171 |
| 7 | 40,260 | 139,339 | 1,075 | 7 | 40,260 | 88,720 | 1,381 | 0.3057 |
| 8 | 40,260 | 141,209 | 1,065 | 8 | 40,260 | 87,297 | 1,387 | 0.3213 |
| 9 | 40,260 | 140,696 | 1,068 | 9 | 40,260 | 84,913 | 1,396 | 0.3281 |
| 10 | 40,260 | 141,324 | 1,065 | 10 | 40,260 | 84,221 | 1,399 | 0.3342 |
| 11 | 40,260 | 142,257 | 1,060 | 11 | 40,260 | 86,958 | 1,388 | 0.3282 |
| 12 | 40,260 | 141,157 | 1,066 | 12 | 40,260 | 87,646 | 1,385 | 0.3197 |
| 13 | 40,260 | 139,441 | 1,075 | 13 | 40,260 | 88,933 | 1,380 | 0.3054 |
| 14 | 40,260 | 139,162 | 1,076 | 14 | 40,260 | 87,974 | 1,384 | 0.3078 |
| 15 | 40,260 | 144,529 | 1,047 | 15 | 40,260 | 87,375 | 1,386 | 0.3390 |
| 16 | 40,260 | 212,529 | 0 | 16 | 40,260 | 171,052 |  |  |
|  |  |  |  |  |  |  | average | 32.52% |

Table B.9 CT Numbers of Slices for  Core#8 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8w | area | 100 % water saturated mean | CT# | 8d | area | 100 % decane saturated mean | CT# |
| 1 | 40,260 | 169,219 | 600 | 1 | 40,260 | 172,707 | 610 |
| 2 | 40,260 | 87,552 | 1,386 | 2 | 40,260 | 102,303 | 1,100 |
| 3 | 40,260 | 83,939 | 1,400 | 3 | 40,260 | 96,680 | 1,130 |
| 4 | 40,260 | 87,376 | 1,386 | 4 | 40,260 | 98,655 | 1,119 |
| 5 | 40,260 | 86,882 | 1,388 | 5 | 40,260 | 99,409 | 1,115 |
| 6 | 40,260 | 87,918 | 1,384 | 6 | 40,260 | 99,947 | 1,112 |
| 7 | 40,260 | 88,720 | 1,381 | 7 | 40,260 | 99,439 | 1,115 |
| 8 | 40,260 | 87,297 | 1,387 | 8 | 40,260 | 99,354 | 1,116 |
| 9 | 40,260 | 84,913 | 1,396 | 9 | 40,260 | 96,924 | 1,129 |
| 10 | 40,260 | 84,221 | 1,399 | 10 | 40,260 | 97,069 | 1,128 |
| 11 | 40,260 | 86,958 | 1,388 | 11 | 40,260 | 97,784 | 1,124 |
| 12 | 40,260 | 87,646 | 1,385 | 12 | 40,260 | 98,440 | 1,120 |
| 13 | 40,260 | 88,933 | 1,380 | 13 | 40,260 | 101,023 | 1,107 |
| 14 | 40,260 | 87,974 | 1,384 | 14 | 40,260 | 98,520 | 1,120 |
| 15 | 40,260 | 87,375 | 1,386 | 15 | 40,260 | 99,039 | 1,117 |
| 16 | 40,260 | 171,052 | 610 | 16 | 40,260 | 173,334 | 611 |

Table B.10 CT Numbers of Slices for  Core#8 Saturation Calculation.

| 8wd | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 168,308 | 600 |  |  |
| 2 | 40,260 | 89,328 | 1,285 | 65% | 35% |
| 3 | 40,260 | 88,333 | 1,283 | 57% | 43% |
| 4 | 40,260 | 94,578 | 1,238 | 45% | 55% |
| 5 | 40,260 | 94,130 | 1,242 | 47% | 53% |
| 6 | 40,260 | 95,203 | 1,233 | 44% | 56% |
| 7 | 40,260 | 96,805 | 1,218 | 39% | 61% |
| 8 | 40,260 | 94,761 | 1,237 | 45% | 55% |
| 9 | 40,260 | 91,735 | 1,264 | 50% | 50% |
| 10 | 40,260 | 91,577 | 1,265 | 51% | 49% |
| 11 | 40,260 | 94,540 | 1,239 | 43% | 57% |
| 12 | 40,260 | 93,979 | 1,244 | 47% | 53% |
| 13 | 40,260 | 95,741 | 1,228 | 44% | 56% |
| 14 | 40,260 | 94,404 | 1,240 | 45% | 55% |
| 15 | 40,260 | 93,562 | 1,247 | 48% | 52% |
| 16 | 40,260 | 170,281 | 605 |  |  |
|  |  |  |  | 47.82% | 52.18% |

Table B.11 CT Numbers of Slices for  Core#8 Saturation Calculation.

| 8wdw | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 171,636 | 615 |  |  |
| 2 | 40,260 | 82,397 | 1,360 | 91% | 9% |
| 3 | 40,260 | 81,279 | 1,372 | 90% | 10% |
| 4 | 40,260 | 86,439 | 1,317 | 74% | 26% |
| 5 | 40,260 | 87,407 | 1,306 | 70% | 30% |
| 6 | 40,260 | 88,750 | 1,292 | 66% | 34% |
| 7 | 40,260 | 90,095 | 1,277 | 61% | 39% |
| 8 | 40,260 | 89,372 | 1,285 | 63% | 37% |
| 9 | 40,260 | 87,136 | 1,309 | 68% | 32% |
| 10 | 40,260 | 87,393 | 1,306 | 66% | 34% |
| 11 | 40,260 | 90,025 | 1,278 | 58% | 42% |
| 12 | 40,260 | 90,371 | 1,274 | 58% | 42% |
| 13 | 40,260 | 92,011 | 1,257 | 55% | 45% |
| 14 | 40,260 | 90,532 | 1,273 | 58% | 42% |
| 15 | 40,260 | 90,308 | 1,275 | 59% | 41% |
| 16 | 40,260 | 170,586 | 605 |  |  |
|  |  |  |  | 66.83% | 33.17% |

## Initially Decane Saturated Water FloodedCT Numbers of Core#8

Table B.12 CT Numbers of Slices for  Core#8 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 81 dry | area | dry mean | CT# | 82 water | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,260 | 182,012 | 580 | 1 | 40,260 | 169,219 | 600 |  |
| 2 | 40,260 | 144,024 | 1,050 | 2 | 40,260 | 87,552 | 1,386 | 0.3355 |
| 3 | 40,260 | 143,745 | 1,052 | 3 | 40,260 | 83,939 | 1,400 | 0.3482 |
| 4 | 40,260 | 143,968 | 1,051 | 4 | 40,260 | 87,376 | 1,386 | 0.3356 |
| 5 | 40,260 | 141,963 | 1,061 | 5 | 40,260 | 86,882 | 1,388 | 0.3270 |
| 6 | 40,260 | 140,872 | 1,067 | 6 | 40,260 | 87,918 | 1,384 | 0.3171 |
| 7 | 40,260 | 139,339 | 1,075 | 7 | 40,260 | 88,720 | 1,381 | 0.3057 |
| 8 | 40,260 | 141,209 | 1,065 | 8 | 40,260 | 87,297 | 1,387 | 0.3213 |
| 9 | 40,260 | 140,696 | 1,068 | 9 | 40,260 | 84,913 | 1,396 | 0.3281 |
| 10 | 40,260 | 141,324 | 1,065 | 10 | 40,260 | 84,221 | 1,399 | 0.3342 |
| 11 | 40,260 | 142,257 | 1,060 | 11 | 40,260 | 86,958 | 1,388 | 0.3282 |
| 12 | 40,260 | 141,157 | 1,066 | 12 | 40,260 | 87,646 | 1,385 | 0.3197 |
| 13 | 40,260 | 139,441 | 1,075 | 13 | 40,260 | 88,933 | 1,380 | 0.3054 |
| 14 | 40,260 | 139,162 | 1,076 | 14 | 40,260 | 87,974 | 1,384 | 0.3078 |
| 15 | 40,260 | 144,529 | 1,047 | 15 | 40,260 | 87,375 | 1,386 | 0.3390 |
| 16 | 40,260 | 212,529 | 0 | 16 | 40,260 | 171,052 | 610 |  |
|  |  |  |  |  |  |  | average  | 32.52% |

Table B.13 CT Numbers of Slices for Core#8 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8w | area | 100 % water saturated mean | CT# | 8d | area | 100 % decane saturated mean | CT# |
| 1 | 40,260 | 169,219 | 600 | 1 | 40,260 | 172,707 | 610 |
| 2 | 40,260 | 87,552 | 1,386 | 2 | 40,260 | 102,303 | 1,100 |
| 3 | 40,260 | 83,939 | 1,400 | 3 | 40,260 | 96,680 | 1,130 |
| 4 | 40,260 | 87,376 | 1,386 | 4 | 40,260 | 98,655 | 1,119 |
| 5 | 40,260 | 86,882 | 1,388 | 5 | 40,260 | 99,409 | 1,115 |
| 6 | 40,260 | 87,918 | 1,384 | 6 | 40,260 | 99,947 | 1,112 |
| 7 | 40,260 | 88,720 | 1,381 | 7 | 40,260 | 99,439 | 1,115 |
| 8 | 40,260 | 87,297 | 1,387 | 8 | 40,260 | 99,354 | 1,116 |
| 9 | 40,260 | 84,913 | 1,396 | 9 | 40,260 | 96,924 | 1,129 |
| 10 | 40,260 | 84,221 | 1,399 | 10 | 40,260 | 97,069 | 1,128 |
| 11 | 40,260 | 86,958 | 1,388 | 11 | 40,260 | 97,784 | 1,124 |
| 12 | 40,260 | 87,646 | 1,385 | 12 | 40,260 | 98,440 | 1,120 |
| 13 | 40,260 | 88,933 | 1,380 | 13 | 40,260 | 101,023 | 1,107 |
| 14 | 40,260 | 87,974 | 1,384 | 14 | 40,260 | 98,520 | 1,120 |
| 15 | 40,260 | 87,375 | 1,386 | 15 | 40,260 | 99,039 | 1,117 |
| 16 | 40,260 | 171,052 | 610 | 16 | 40,260 | 173,334 | 611 |

Table B.14 CT Numbers of Slices for Core#8 Saturation Calculation.

| 8wdw | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 171,636 | 615 |  |  |
| 2 | 40,260 | 82,397 | 1,360 | 91% | 9% |
| 3 | 40,260 | 81,279 | 1,372 | 90% | 10% |
| 4 | 40,260 | 86,439 | 1,317 | 86% | 14% |
| 5 | 40,260 | 87,407 | 1,306 | 70% | 30% |
| 6 | 40,260 | 88,750 | 1,292 | 66% | 34% |
| 7 | 40,260 | 90,095 | 1,277 | 61% | 39% |
| 8 | 40,260 | 89,372 | 1,285 | 63% | 37% |
| 9 | 40,260 | 87,136 | 1,309 | 68% | 32% |
| 10 | 40,260 | 87,393 | 1,306 | 66% | 34% |
| 11 | 40,260 | 90,025 | 1,278 | 58% | 42% |
| 12 | 40,260 | 90,371 | 1,274 | 58% | 42% |
| 13 | 40,260 | 92,011 | 1,257 | 55% | 45% |
| 14 | 40,260 | 90,532 | 1,273 | 58% | 42% |
| 15 | 40,260 | 90,308 | 1,275 | 59% | 41% |
| 16 | 40,260 | 170,586 | 605 |  |  |
|  |  |  |  | 67.69% | 32.31% |

# APPENDIX-D

## Initially Water Saturated Decane Flooded Artificially Fractured CT Numbers of Core#3

Table D.1 CT Numbers of Slices for Core#3 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3afm1 dry | Area | dry mean | CT# | 3afm2 water | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,258 | 221,217 | -10 | 1 | 40,258 | 208,489 | -11 |  |
| 2 | 40,258 | 140,989 | 1,007 | 2 | 40,258 | 78,713 | 1,306 | 0.2996 |
| 3 | 40,258 | 140,878 | 1,008 | 3 | 40,258 | 69,543 | 1,474 | 0.4666 |
| 4 | 40,258 | 139,605 | 1,023 | 4 | 40,258 | 75,311 | 1,369 | 0.3456 |
| 5 | 40,258 | 137,445 | 1,049 | 5 | 40,258 | 75,574 | 1,364 | 0.3150 |
| 6 | 40,258 | 134,679 | 1,082 | 6 | 40,258 | 74,266 | 1,388 | 0.3061 |
| 7 | 40,258 | 135,160 | 1,076 | 7 | 40,258 | 77,758 | 1,324 | 0.2478 |
| 8 | 40,258 | 135,419 | 1,073 | 8 | 40,258 | 67,236 | 1,517 | 0.4439 |
| 9 | 40,258 | 136,777 | 1,057 | 9 | 40,258 | 73,729 | 1,398 | 0.3409 |
| 10 | 40,258 | 139,962 | 1,019 | 10 | 40,258 | 71,341 | 1,441 | 0.4227 |
| 11 | 40,258 | 140,290 | 1,015 | 11 | 40,258 | 70,916 | 1,449 | 0.4344 |
| 12 | 40,258 | 146,580 | 940 | 12 | 40,258 | 78,095 | 1,317 | 0.3775 |
| 13 | 40,258 | 142,127 | 993 | 13 | 40,258 | 80,752 | 1,269 | 0.2757 |
| 14 | 40,258 | 139,480 | 1,024 | 14 | 40,258 | 75,632 | 1,363 | 0.3382 |
| 15 | 40,258 | 142,091 | 993 | 15 | 40,258 | 85,587 | 1,180 | 0.1866 |
| 16 | 40,258 | 210,355 | 0 | 16 | 40,258 | 209,821 | 1,100 |  |
|  |  |  |  |  |  |  | average  | 34.29% |

Table D.2 CT Numbers of Slices for Core#3 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3afmw1 | area | water 100 % saturated mean | CT# | 3afmd2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,258 | 208,489 | -11 | 1 | 40,258 | 172,707 | 615 |
| 2 | 40,258 | 78,713 | 1,306 | 2 | 40,258 | 102,303 | 1,085 |
| 3 | 40,258 | 69,543 | 1,474 | 3 | 40,258 | 96,680 | 1,115 |
| 4 | 40,258 | 75,311 | 1,369 | 4 | 40,258 | 98,655 | 1,104 |
| 5 | 40,258 | 75,574 | 1,364 | 5 | 40,258 | 99,409 | 1,100 |
| 6 | 40,258 | 74,266 | 1,388 | 6 | 40,258 | 99,947 | 1,097 |
| 7 | 40,258 | 77,758 | 1,324 | 7 | 40,258 | 99,439 | 1,100 |
| 8 | 40,258 | 67,236 | 1,517 | 8 | 40,258 | 99,354 | 1,101 |
| 9 | 40,258 | 73,729 | 1,398 | 9 | 40,258 | 96,924 | 1,114 |
| 10 | 40,258 | 71,341 | 1,441 | 10 | 40,258 | 97,069 | 1,113 |
| 11 | 40,258 | 70,916 | 1,449 | 11 | 40,258 | 97,784 | 1,109 |
| 12 | 40,258 | 78,095 | 1,317 | 12 | 40,258 | 98,440 | 1,105 |
| 13 | 40,258 | 80,752 | 1,269 | 13 | 40,258 | 101,023 | 1,092 |
| 14 | 40,258 | 75,632 | 1,363 | 14 | 40,258 | 98,520 | 1,105 |
| 15 | 40,258 | 85,587 | 1,180 | 15 | 40,258 | 99,039 | 1,102 |
| 16 | 40,258 | 209,821 | 1,100 | 16 | 40,258 | 173,334 | 616 |

Table D.3 CT Numbers of Slices for Core#3 Saturation Calculation.

| 3afmwd2 | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 164,445 | 550 |  |  |
| 2 | 40,258 | 74,162 | 1,249 | 74% | 26% |
| 3 | 40,258 | 67,243 | 1,230 | 32% | 68% |
| 4 | 40,258 | 68,009 | 1,224 | 45% | 55% |
| 5 | 40,258 | 68,619 | 1,228 | 48% | 52% |
| 6 | 40,258 | 67,733 | 1,230 | 46% | 54% |
| 7 | 40,258 | 69,973 | 1,219 | 53% | 47% |
| 8 | 40,258 | 65,022 | 1,241 | 34% | 66% |
| 9 | 40,258 | 66,52 | 1,236 | 43% | 57% |
| 10 | 40,258 | 67,954 | 1,232 | 36% | 64% |
| 11 | 40,258 | 67,538 | 1,228 | 35% | 65% |
| 12 | 40,258 | 72,462 | 1,210 | 49% | 51% |
| 13 | 40,258 | 73,167 | 1,210 | 67% | 33% |
| 14 | 40,258 | 70,655 | 1,219 | 44% | 56% |
| 15 | 40,258 | 82,803 | 1,160 | 74% | 26% |
| 16 | 40,258 | 169,473 | 400 |  |  |
|  |  |  |  | 48.71% | 51.29% |

Table D.4 CT Numbers of Slices for Core#3 Saturation Calculation.

| 3afmwd2 | area | water after decane and gel mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 207,489 | 1,200 |  |  |
| 2 | 40,258 | 78,115 | 1,224 | 63% | 37% |
| 3 | 40,258 | 66,994 | 1,347 | 64% | 36% |
| 4 | 40,258 | 68,258 | 1,333 | 87% | 13% |
| 5 | 40,258 | 67,361 | 1,322 | 84% | 16% |
| 6 | 40,258 | 66,854 | 1,338 | 83% | 17% |
| 7 | 40,258 | 69,302 | 1,298 | 89% | 11% |
| 8 | 40,258 | 64,447 | 1,386 | 69% | 31% |
| 9 | 40,258 | 65,473 | 1,360 | 87% | 13% |
| 10 | 40,258 | 66,311 | 1,334 | 67% | 33% |
| 11 | 40,258 | 67,294 | 1,341 | 68% | 32% |
| 12 | 40,258 | 71,475 | 1,254 | 70% | 30% |
| 13 | 40,258 | 71,522 | 1,241 | 85% | 15% |
| 14 | 40,258 | 69,361 | 1,286 | 70% | 30% |
| 15 | 40,258 | 82,977 | 1,070 | 20% | 80% |
| 16 | 40,258 | 208,821 | 1,120 |  |  |
|  |  |  |  | 71.78% | 28.22% |

Initially Water Saturated Decane Flooded Artificially Fractured CT Numbers of Core#3 Left Matrix

Table D.5 CT Numbers of Slices for Core#3 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3lm1 dry | area | dry mean | CT# | 3lm 2 | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,258 | 221,217 | -10 | 1 | 40,258 | 208,489 | -11 |  |
| 2 | 17,250 | 140,989 | 1,032 | 2 | 17,250 | 78,713 | 1,329 | 0.2976 |
| 3 | 17,250 | 140,878 | 1,033 | 3 | 17,250 | 69,543 | 1,497 | 0.4646 |
| 4 | 17,250 | 139,605 | 1,048 | 4 | 17,250 | 75,311 | 1,392 | 0.3436 |
| 5 | 17,250 | 137,445 | 1,074 | 5 | 17,250 | 75,574 | 1,387 | 0.3130 |
| 6 | 17,250 | 134,679 | 1,107 | 6 | 17,250 | 74,266 | 1,411 | 0.3041 |
| 7 | 17,250 | 135,160 | 1,101 | 7 | 17,250 | 77,758 | 1,347 | 0.2458 |
| 8 | 17,250 | 135,419 | 1,098 | 8 | 17,250 | 67,236 | 1,540 | 0.4419 |
| 9 | 17,250 | 136,777 | 1,082 | 9 | 17,250 | 73,729 | 1,421 | 0.3389 |
| 10 | 17,250 | 139,962 | 1,044 | 10 | 17,250 | 71,341 | 1,464 | 0.4207 |
| 11 | 17,250 | 140,290 | 1,040 | 11 | 17,250 | 70,916 | 1,472 | 0.4324 |
| 12 | 17,250 | 146,580 | 965 | 12 | 17,250 | 78,095 | 1,340 | 0.3755 |
| 13 | 17,250 | 142,127 | 1,018 | 13 | 17,250 | 80,752 | 1,292 | 0.2737 |
| 14 | 17,250 | 139,480 | 1,049 | 14 | 17,250 | 75,632 | 1,386 | 0.3362 |
| 15 | 17,250 | 142,091 | 1,018 | 15 | 17,250 | 85,587 | 1,203 | 0.1846 |
| 16 | 40,258 | 210,355 | 0 | 16 | 40,258 | 209,821 | 1,100 |  |
|  |  |  |  |  |  |  | Average  | 34.09% |

Table D.6 CT Numbers of Slices for Core#3 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3lmw1 | area | 100 % water saturated mean | CT# | 3lmd2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,258 | 208,489 | -11 | 1 | 40,258 | 172,707 | 615 |
| 2 | 17,250 | 78,713 | 10329 | 2 | 17,250 | 102,303 | 1,085 |
| 3 | 17,250 | 69,543 | 1,497 | 3 | 17,250 | 96,680 | 1,115 |
| 4 | 17,250 | 75,311 | 1,392 | 4 | 17,250 | 98,655 | 1,104 |
| 5 | 17,250 | 75,574 | 1,387 | 5 | 17,250 | 99,409 | 1,100 |
| 6 | 17,250 | 74,266 | 1,411 | 6 | 17,250 | 99,947 | 1,097 |
| 7 | 17,250 | 77,758 | 1,347 | 7 | 17,250 | 99,439 | 1,100 |
| 8 | 17,250 | 67,236 | 1,540 | 8 | 17,250 | 99,354 | 1,101 |
| 9 | 17,250 | 73,729 | 1,421 | 9 | 17,250 | 96,924 | 1,114 |
| 10 | 17,250 | 71,341 | 1,464 | 10 | 17,250 | 97,069 | 1,113 |
| 11 | 17,250 | 70,916 | 1,472 | 11 | 17,250 | 97,784 | 1,109 |
| 12 | 17,250 | 78,095 | 1,340 | 12 | 17,250 | 98,440 | 1,105 |
| 13 | 17,250 | 80,752 | 1,292 | 13 | 17,250 | 101,023 | 1,092 |
| 14 | 17,250 | 75,632 | 1,386 | 14 | 17,250 | 98,520 | 1,105 |
| 15 | 17,250 | 85,587 | 1,203 | 15 | 17,250 | 99,039 | 1,102 |
| 16 | 40,258 | 209,821 | 1,100 | 16 | 40,258 | 173,334 | 616 |

Table D.7 CT Numbers of Slices for Core#3 Saturation Calculation.

| 3lmwd2 | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 164,445 | 550 |  |  |
| 2 | 17,250 | 74,162 | 1,259 | 71% | 29% |
| 3 | 17,250 | 67,243 | 1,240 | 33% | 67% |
| 4 | 17,250 | 68,009 | 1,234 | 45% | 55% |
| 5 | 17,250 | 68,619 | 1,238 | 48% | 52% |
| 6 | 17,250 | 67,733 | 1,240 | 46% | 54% |
| 7 | 17,250 | 69,973 | 1,229 | 52% | 48% |
| 8 | 17,250 | 65,022 | 1,251 | 34% | 66% |
| 9 | 17,250 | 66,52 | 1,246 | 43% | 57% |
| 10 | 17,250 | 67,954 | 1,242 | 37% | 63% |
| 11 | 17,250 | 67,538 | 1,238 | 36% | 64% |
| 12 | 17,250 | 72,462 | 1,220 | 49% | 51% |
| 13 | 17,250 | 73,167 | 1,220 | 64% | 36% |
| 14 | 17,250 | 70,655 | 1,229 | 44% | 56% |
| 15 | 17,250 | 82,803 | 1,170 | 67% | 33% |
| 16 | 40,258 | 169,473 | 400 |  |  |
|  |  |  |  | 47.80% | 52.20% |

Table D.8 CT Numbers of Slices for Core#3 Saturation Calculation.

| 3lmwdw3 | area | water after decane and gel | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 207,489 | 1,200 |  |  |
| 2 | 17,250 | 78,115 | 1,224 | 57% | 43% |
| 3 | 17,250 | 66,994 | 1,347 | 61% | 39% |
| 4 | 17,250 | 68,258 | 1,333 | 80% | 20% |
| 5 | 17,250 | 67,361 | 1,322 | 77% | 23% |
| 6 | 17,250 | 66,854 | 1,338 | 77% | 23% |
| 7 | 17,250 | 69,302 | 1,298 | 80% | 20% |
| 8 | 17,250 | 64,447 | 1,386 | 65% | 35% |
| 9 | 17,250 | 65,473 | 1,360 | 80% | 20% |
| 10 | 17,250 | 66,311 | 1,334 | 63% | 37% |
| 11 | 17,250 | 67,294 | 1,341 | 64% | 36% |
| 12 | 17,250 | 71,475 | 1,254 | 63% | 37% |
| 13 | 17,250 | 71,522 | 1,241 | 75% | 25% |
| 14 | 17,250 | 69,361 | 1,286 | 64% | 36% |
| 15 | 17,250 | 82,977 | 1,070 | 62% | 132% |
| 16 | 40,258 | 208,821 | 1,120 |  |  |
|  |  |  |  | 62.42% | 37.58% |

Initially Water Saturated Decane Flooded Artificially Fractured CT Numbers of Core#3 Right Matrix

Table D.9 CT Numbers of Slices for Core#3 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3rm1 dry | area | dry mean | CT# | 3rm2 | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,258 | 221,217 | -10 | 1 | 40,258 | 208,489 | -11 |  |
| 2 | 17,250 | 143,843 | 1,017 | 2 | 17,250 | 76,147 | 1,390 | 0.3737 |
| 3 | 17,250 | 136,792 | 1,131 | 3 | 17,250 | 72,475 | 1,429 | 0.2985 |
| 4 | 17,250 | 139,122 | 1,093 | 4 | 17,250 | 72,657 | 1,428 | 0.3344 |
| 5 | 17,250 | 140,736 | 1,067 | 5 | 17,250 | 67,892 | 1,478 | 0.4115 |
| 6 | 17,250 | 139,463 | 1,088 | 6 | 17,250 | 64,835 | 1,511 | 0.4235 |
| 7 | 17,250 | 147,242 | 961 | 7 | 17,250 | 70,708 | 1,448 | 0.4870 |
| 8 | 17,250 | 133,978 | 1,177 | 8 | 17,250 | 65,476 | 1,504 | 0.3276 |
| 9 | 17,250 | 131,410 | 1,218 | 9 | 17,250 | 69,464 | 1,462 | 0.2434 |
| 10 | 17,250 | 132,814 | 1,196 | 10 | 17,250 | 70,380 | 1,452 | 0.2564 |
| 11 | 17,250 | 132,790 | 1,196 | 11 | 17,250 | 69,929 | 1,457 | 0.2608 |
| 12 | 17,250 | 137,073 | 1,126 | 12 | 17,250 | 75,933 | 1,393 | 0.2662 |
| 13 | 17,250 | 147,636 | 955 | 13 | 17,250 | 73,006 | 1,424 | 0.4688 |
| 14 | 17,250 | 138,731 | 1,099 | 14 | 17,250 | 73,224 | 1,421 | 0.3220 |
| 15 | 17,250 | 141,189 | 1,060 | 15 | 17,250 | 75,517 | 1,397 | 0.3374 |
| 16 | 40,258 | 210,355 | 0 | 16 | 40,258 | 209,821 | 0 |  |
|  |  |  |  |  |  |  | average  | 34.37% |

Table D.10 CT Numbers of Slices for Core#3 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3rmw1 | area | 100 % water saturated mean | CT# | 3rmd2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,258 | 208,489 | -11 | 1 | 40,258 | 172,707 | 615 |
| 2 | 17,250 | 76,147 | 1,390 | 2 | 17,250 | 102,303 | 1,085 |
| 3 | 17,250 | 72,475 | 1,429 | 3 | 17,250 | 96,680 | 1,115 |
| 4 | 17,250 | 72,657 | 1,428 | 4 | 17,250 | 98,655 | 1,104 |
| 5 | 17,250 | 67,892 | 1,478 | 5 | 17,250 | 99,409 | 1,100 |
| 6 | 17,250 | 64,835 | 1,511 | 6 | 17,250 | 99,947 | 1,097 |
| 7 | 17,250 | 70,708 | 1,448 | 7 | 17,250 | 99,439 | 1,100 |
| 8 | 17,250 | 65,476 | 1,504 | 8 | 17,250 | 99,354 | 1,101 |
| 9 | 17,250 | 69,464 | 1,462 | 9 | 17,250 | 96,924 | 1,114 |
| 10 | 17,250 | 70,38 | 1,452 | 10 | 17,250 | 97,069 | 1,113 |
| 11 | 17,250 | 69,929 | 1,457 | 11 | 17,250 | 97,784 | 1,109 |
| 12 | 17,250 | 75,933 | 1,393 | 12 | 17,250 | 98,440 | 1,105 |
| 13 | 17,250 | 73,006 | 1,424 | 13 | 17,250 | 101,023 | 1,092 |
| 14 | 17,250 | 73,224 | 1,421 | 14 | 17,250 | 98,520 | 1,105 |
| 15 | 17,250 | 75,517 | 1,397 | 15 | 17,250 | 99,039 | 1,102 |
| 16 | 40,258 | 209,821 | 1,100 | 16 | 40,258 | 173,334 | 616 |

Table D.11 CT Numbers of Slices for Core#3 Saturation Calculation.

| 3rmwd2 |  | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 164,445 | 550 |  |  |
| 2 | 17,250 | 73,662 | 1,120 | 11% | 89% |
| 3 | 17,250 | 66,743 | 1,324 | 66% | 34% |
| 4 | 17,250 | 67,509 | 1,271 | 52% | 48% |
| 5 | 17,250 | 68,119 | 1,317 | 57% | 43% |
| 6 | 17,250 | 67,233 | 1,327 | 55% | 45% |
| 7 | 17,250 | 69,473 | 1,282 | 52% | 48% |
| 8 | 17,250 | 64,522 | 1,371 | 44% | 56% |
| 9 | 17,250 | 66,02 | 1,352 | 48% | 52% |
| 10 | 17,250 | 67,454 | 1,337 | 55% | 45% |
| 11 | 17,250 | 67,038 | 1,319 | 45% | 55% |
| 12 | 17,250 | 71,962 | 1,242 | 48% | 52% |
| 13 | 17,250 | 72,667 | 1,241 | 45% | 55% |
| 14 | 17,250 | 70,155 | 1,281 | 55% | 45% |
| 15 | 17,250 | 82,303 | 1,031 | 45% | 55% |
| 16 | 40,258 | 169,473 | 400 |  |  |
|  |  |  |  | 48.56% | 51.44% |

Table D.12 CT Numbers of Slices for Core#3 Saturation Calculation.

| 3rmwdw3 | area | water after decane and gel mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 207,489 | 1,200 |  |  |
| 2 | 17,250 | 78,365 | 1,249 | 54% | 46% |
| 3 | 17,250 | 67,244 | 1,372 | 82% | 18% |
| 4 | 17,250 | 68,508 | 1,358 | 78% | 22% |
| 5 | 17,250 | 67,611 | 1,347 | 65% | 35% |
| 6 | 17,250 | 67,104 | 1,363 | 64% | 36% |
| 7 | 17,250 | 69,552 | 1,323 | 64% | 36% |
| 8 | 17,250 | 64,697 | 1,411 | 57% | 43% |
| 9 | 17,250 | 65,723 | 1,385 | 58% | 42% |
| 10 | 17,250 | 66,561 | 1,359 | 55% | 45% |
| 11 | 17,250 | 67,544 | 1,366 | 74% | 26% |
| 12 | 17,250 | 71,725 | 1,279 | 60% | 40% |
| 13 | 17,250 | 71,772 | 1,266 | 53% | 47% |
| 14 | 17,250 | 69,611 | 1,311 | 65% | 35% |
| 15 | 17,250 | 83,227 | 1,095 | 55% | 45% |
| 16 | 40,258 | 208,821 | 1,120 |  |  |
|  |  |  |  | 63.16% | 36.84% |

## Initially Water Saturated Decane Flooded Artificialy Fractured CT Scan Images of Core#7

Figure D.1 CT Slice Views of Core#7 Water Saturated.

Figure D.2 CT Slice Views of Artificially Fractured Core#7 Water Saturated.

Figure D.3 CT Slice Views of Artificially Fractured Core#7 Decane Flooded.

Figure D.4 CT Slice Views of Artificially Fractured Core#7 Water Flooded for Production.

Figure D.5 CT Slice Views of  Artificially Fractured Core#7 Polymer Gel Injected then Produced.

## Initially Water Saturated Decane Flooded Artificially Fractured CT Numbers of Core#7

Table D.13 CT Numbers of Slices for Core#7 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7af0 dry | area | mean | CT# | 7af1 | area | water | CT# | fraction |
| 1 | 40,258 | 195,133 | 50 | 1 | 40,258 | 204,117 | -70 |  |
| 2 | 40,258 | 133,002 | 1,056 | 2 | 40,258 | 80,570 | 1,370 | 0.3145 |
| 3 | 40,258 | 134,323 | 1,050 | 3 | 40,258 | 80,539 | 1,371 | 0.3209 |
| 4 | 40,258 | 135,310 | 1,046 | 4 | 40,258 | 81,655 | 1,370 | 0.3240 |
| 5 | 40,258 | 135,260 | 1,046 | 5 | 40,258 | 82,859 | 1,369 | 0.3228 |
| 6 | 40,258 | 131,889 | 1,060 | 6 | 40,258 | 78,140 | 1,371 | 0.3109 |
| 7 | 40,258 | 132,634 | 1,057 | 7 | 40,258 | 79,406 | 1,373 | 0.3160 |
| 8 | 40,258 | 133,829 | 1,052 | 8 | 40,258 | 79,664 | 1,373 | 0.3209 |
| 9 | 40,258 | 133,260 | 1,054 | 9 | 40,258 | 81,393 | 1,372 | 0.3175 |
| 10 | 40,258 | 135,267 | 1,046 | 10 | 40,258 | 83,201 | 1,373 | 0.3269 |
| 11 | 40,258 | 135,969 | 1,043 | 11 | 40,258 | 85,055 | 1,373 | 0.3298 |
| 12 | 40,258 | 134,160 | 1,051 | 12 | 40,258 | 84,065 | 1,372 | 0.3213 |
| 13 | 40,258 | 133,863 | 1,052 | 13 | 40,258 | 82,724 | 1,372 | 0.3200 |
| 14 | 40,258 | 134,737 | 1,048 | 14 | 40,258 | 84,938 | 1,371 | 0.3227 |
| 15 | 40,258 | 136,028 | 1,043 | 15 | 40,258 | 85,893 | 1,381 | 0.3384 |
| 16 | 40,258 | 243,090 | 0 | 16 | 40,258 | 229,592 | 50 |  |
|  |  |  |  |  |  |  | average | 32.14% |

Table D.14 CT Numbers of Slices for Core#7 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7afw0 | area | 100 % water saturated mean | CT# | 7afd1 | area | 100 % decane saturated mean | CT# |
| 1 | 40,258 | 204,117 | -70 | 1 | 40,258 | 172,707 | 630 |
| 2 | 40,258 | 80,570 | 1,370 | 2 | 40,258 | 102,303 | 1,185 |
| 3 | 40,258 | 80,539 | 1,371 | 3 | 40,258 | 96,68 | 1,215 |
| 4 | 40,258 | 81,655 | 1,370 | 4 | 40,258 | 98,655 | 1,204 |
| 5 | 40,258 | 82,859 | 1,369 | 5 | 40,258 | 99,409 | 1,200 |
| 6 | 40,258 | 78,14 | 1,371 | 6 | 40,258 | 99,947 | 1,197 |
| 7 | 40,258 | 79,406 | 1,373 | 7 | 40,258 | 99,439 | 1,200 |
| 8 | 40,258 | 79,664 | 1,373 | 8 | 40,258 | 99,354 | 1,201 |
| 9 | 40,258 | 81,393 | 1,372 | 9 | 40,258 | 96,924 | 1,214 |
| 10 | 40,258 | 83,201 | 1,373 | 10 | 40,258 | 97,069 | 1,213 |
| 11 | 40,258 | 85,055 | 1,373 | 11 | 40,258 | 97,784 | 1,209 |
| 12 | 40,258 | 84,065 | 1,372 | 12 | 40,258 | 98,440 | 1,205 |
| 13 | 40,258 | 82,724 | 1,372 | 13 | 40,258 | 101,023 | 1,192 |
| 14 | 40,258 | 84,938 | 1,371 | 14 | 40,258 | 98,520 | 1,205 |
| 15 | 40,258 | 85,893 | 1,381 | 15 | 40,258 | 99,039 | 1,202 |
| 16 | 40,258 | 229,592 | 1,100 | 16 | 40,258 | 173,334 | 631 |

Table D.15 CT Numbers of Slices for Core#7 Saturation Calculation.

| 7afwd1 | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 190,462 | -11 |  |  |
| 2 | 40,258 | 67,338 | 1,259 | 40% | 60% |
| 3 | 40,258 | 61,774 | 1,249 | 22% | 78% |
| 4 | 40,258 | 62,950 | 1,267 | 38% | 62% |
| 5 | 40,258 | 63,594 | 1,257 | 34% | 66% |
| 6 | 40,258 | 63,098 | 1,264 | 39% | 61% |
| 7 | 40,258 | 66,680 | 1,276 | 44% | 56% |
| 8 | 40,258 | 65,031 | 1,297 | 35% | 65% |
| 9 | 40,258 | 66,303 | 1,277 | 40% | 60% |
| 10 | 40,258 | 64,775 | 1,300 | 38% | 62% |
| 11 | 40,258 | 65,540 | 1,297 | 51% | 49% |
| 12 | 40,258 | 68,379 | 1,289 | 50% | 50% |
| 13 | 40,258 | 69,054 | 1,287 | 43% | 57% |
| 14 | 40,258 | 69,789 | 1,288 | 50% | 50% |
| 15 | 40,258 | 76,669 | 1,204 | 51% | 49% |
| 16 | 40,258 | 169,473 | 400 |  |  |
|  |  |  |  | 41.00% | 59.00% |

Table D.16 CT Numbers of Slices for  Core#7 Saturation Calculation.

| 7afwd2 | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 218,127 | 650 |  |  |
| 2 | 40,258 | 76,305 | 1,220 | 100% | 0% |
| 3 | 40,258 | 73,931 | 1,240 | 75% | 25% |
| 4 | 40,258 | 76,222 | 1,220 | 64% | 36% |
| 5 | 40,258 | 77,578 | 1,208 | 88% | 12% |
| 6 | 40,258 | 76,527 | 1,218 | 66% | 34% |
| 7 | 40,258 | 73,950 | 1,240 | 48% | 52% |
| 8 | 40,258 | 72,232 | 1,255 | 46% | 54% |
| 9 | 40,258 | 74,173 | 1,238 | 70% | 30% |
| 10 | 40,258 | 72,061 | 1,257 | 31% | 69% |
| 11 | 40,258 | 71,703 | 1,260 | 44% | 56% |
| 12 | 40,258 | 72,665 | 1,252 | 49% | 51% |
| 13 | 40,258 | 71,870 | 1,259 | 45% | 55% |
| 14 | 40,258 | 72,376 | 1,254 | 44% | 56% |
| 15 | 40,258 | 80,512 | 1,183 | 45% | 55% |
| 16 | 40,258 | 239,973 | 450 |  |  |
|  |  |  |  | 58.30% | 41.70% |

Table D.17 CT Numbers of Slices for Core#7 Saturation Calculation after Gel Injection and Production.

| 7afwdwg | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 200,922 | -100 |  |  |
| 2 | 40,258 | 75,676 | 1,305 | 65% | 35% |
| 3 | 40,258 | 77,115 | 1,362 | 94% | 6% |
| 4 | 40,258 | 74,634 | 1,350 | 88% | 12% |
| 5 | 40,258 | 75,959 | 1,343 | 85% | 15% |
| 6 | 40,258 | 74,988 | 1,349 | 87% | 13% |
| 7 | 40,258 | 73,435 | 1,312 | 65% | 35% |
| 8 | 40,258 | 70,556 | 1,329 | 74% | 26% |
| 9 | 40,258 | 73,307 | 1,316 | 64% | 36% |
| 10 | 40,258 | 70,252 | 1,331 | 74% | 26% |
| 11 | 40,258 | 70,678 | 1,323 | 70% | 30% |
| 12 | 40,258 | 71,743 | 1,294 | 53% | 47% |
| 13 | 40,258 | 71,916 | 1,287 | 53% | 47% |
| 14 | 40,258 | 71,839 | 1,280 | 45% | 55% |
| 15 | 40,258 | 83,119 | 1,209 | 60% | 40% |
| 16 | 40,258 | 242,565 | 1,120 |  |  |
|  |  |  |  | 65.82% | 34.18% |

Initially Water Saturated Decane Flooded Artificially Fractured CT Numbers of Core#7 Left Matrix

Table D.18 CT Numbers of Slices for Core#7 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7lm1 dry | area | dry mean | CT# | 7lmw2 | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,258 | 213,168 | -11 | 1 | 40,258 | 214,062 | -10 |  |
| 2 | 17,250 | 122,002 | 1,158 | 2 | 17,250 | 74,825 | 1,453 | 0.2958 |
| 3 | 17,250 | 123,323 | 1,152 | 3 | 17,250 | 71,091 | 1,503 | 0.3508 |
| 4 | 17,250 | 124,310 | 1,148 | 4 | 17,250 | 71,806 | 1,493 | 0.3454 |
| 5 | 17,250 | 124,260 | 1,148 | 5 | 17,250 | 71,984 | 1,491 | 0.3428 |
| 6 | 17,250 | 120,889 | 1,162 | 6 | 17,250 | 72,622 | 1,483 | 0.3204 |
| 7 | 17,250 | 121,634 | 1,159 | 7 | 17,250 | 73,214 | 1,475 | 0.3157 |
| 8 | 17,250 | 122,829 | 1,154 | 8 | 17,250 | 74,319 | 1,460 | 0.3060 |
| 9 | 17,250 | 122,260 | 1,156 | 9 | 17,250 | 73,722 | 1,468 | 0.3115 |
| 10 | 17,250 | 124,267 | 1,148 | 10 | 17,250 | 72,770 | 1,481 | 0.3324 |
| 11 | 17,250 | 124,969 | 1,145 | 11 | 17,250 | 76,808 | 1,427 | 0.2819 |
| 12 | 17,250 | 123,160 | 1,153 | 12 | 17,250 | 70,434 | 1,512 | 0.3588 |
| 13 | 17,250 | 122,863 | 1,154 | 13 | 17,250 | 70,898 | 1,505 | 0.3514 |
| 14 | 17,250 | 123,737 | 1,150 | 14 | 17,250 | 69,389 | 1,525 | 0.3750 |
| 15 | 17,250 | 125,028 | 1,145 | 15 | 17,250 | 82,631 | 1,350 | 0.2050 |
| 16 | 40,258 | 218,805 | -1 | 16 | 40,258 | 218,494 | 1,100 |  |
|  |  |  |  |  |  |  | average  | 32.09% |

Table D.19 CT Numbers of Slices for  Core#7 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7lmw1 | area | 100 % water saturated mean | CT# | 7lmd2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,258 | 214,062 | -10 | 1 | 40,258 | 172,707 | 630 |
| 2 | 17,250 | 74,825 | 1453 | 2 | 17,250 | 102,303 | 1,185 |
| 3 | 17,250 | 71,091 | 1503 | 3 | 17,250 | 96,680 | 1,215 |
| 4 | 17,250 | 71,806 | 1493 | 4 | 17,250 | 98,655 | 1,204 |
| 5 | 17,250 | 71,984 | 1491 | 5 | 17,250 | 99,409 | 1,200 |
| 6 | 17,250 | 72,622 | 1483 | 6 | 17,250 | 99,947 | 1,197 |
| 7 | 17,250 | 73,214 | 1475 | 7 | 17,250 | 99,439 | 1,200 |
| 8 | 17,250 | 74,319 | 1460 | 8 | 17,250 | 99,354 | 1,201 |
| 9 | 17,250 | 73,722 | 1468 | 9 | 17,250 | 96,924 | 1,214 |
| 10 | 17,250 | 72,770 | 1481 | 10 | 17,250 | 97,069 | 1,213 |
| 11 | 17,250 | 76,808 | 1427 | 11 | 17,250 | 97,784 | 1,209 |
| 12 | 17,250 | 70,434 | 1512 | 12 | 17,250 | 98,440 | 1,205 |
| 13 | 17,250 | 70,898 | 1505 | 13 | 17,250 | 101,023 | 1,192 |
| 14 | 17,250 | 69,389 | 1525 | 14 | 17,250 | 98,520 | 1,205 |
| 15 | 17,250 | 82,631 | 1350 | 15 | 17,250 | 99,039 | 1,202 |
| 16 | 40,258 | 218,494 | 1100 | 16 | 40,258 | 173,334 | 631 |

Table D.20 CT Numbers of Slices for Core#7 Saturation Calculation.

| 7lmwd2 | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 190,362 | -11 |  |  |
| 2 | 17,250 | 66,838 | 1,293 | 40% | 60% |
| 3 | 17,250 | 61,274 | 1,274 | 21% | 79% |
| 4 | 17,250 | 62,450 | 1,306 | 35% | 65% |
| 5 | 17,250 | 63,094 | 1,289 | 31% | 69% |
| 6 | 17,250 | 62,598 | 1,302 | 37% | 63% |
| 7 | 17,250 | 66,180 | 1,322 | 44% | 56% |
| 8 | 17,250 | 64,531 | 1,359 | 61% | 39% |
| 9 | 17,250 | 65,803 | 1,323 | 43% | 57% |
| 10 | 17,250 | 64,275 | 1,363 | 56% | 44% |
| 11 | 17,250 | 65,040 | 1,357 | 68% | 32% |
| 12 | 17,250 | 67,879 | 1,344 | 45% | 55% |
| 13 | 17,250 | 68,554 | 1,341 | 48% | 52% |
| 14 | 17,250 | 69,289 | 1,342 | 43% | 57% |
| 15 | 17,250 | 76,169 | 1,210 | 5% | 95% |
| 16 | 40,258 | 216,568 | 50 |  |  |
|  |  |  |  | 41.18% | 58.82% |

Table D.21 CT Numbers of Slices for  Core#7 Saturation Calculation.

| 7lmwd2 | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 218,127 | 650 |  |  |
| 2 | 17,250 | 77,405 | 1,237 | 100% | 0% |
| 3 | 17,250 | 75,031 | 1,258 | 45% | 55% |
| 4 | 17,250 | 77,322 | 1,238 | 52% | 48% |
| 5 | 17,250 | 78,678 | 1,226 | 76% | 24% |
| 6 | 17,250 | 77,627 | 1,235 | 59% | 41% |
| 7 | 17,250 | 75,050 | 1,258 | 43% | 57% |
| 8 | 17,250 | 73,332 | 1,273 | 40% | 60% |
| 9 | 17,250 | 75,273 | 1,256 | 71% | 29% |
| 10 | 17,250 | 73,161 | 1,274 | 75% | 25% |
| 11 | 17,250 | 72,803 | 1,278 | 55% | 45% |
| 12 | 17,250 | 73,765 | 1,269 | 58% | 42% |
| 13 | 17,250 | 72,970 | 1,276 | 44% | 56% |
| 14 | 17,250 | 73,476 | 1,272 | 51% | 49% |
| 15 | 17,250 | 81,612 | 1,200 | 80% | 20% |
| 16 | 40,258 | 239,973 | 450 |  |  |
|  |  |  |  | 60.73% | 39.27% |

Table D.22 CT Numbers of Slices for Core#7 Saturation Calculation after Gel Injection and Production.

| 7lmwdwg | area | water after decane and gel mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 200,922 | -100 |  |  |
| 2 | 17,250 | 75,276 | 1,369 | 69% | 31% |
| 3 | 17,250 | 76,715 | 1,452 | 82% | 18% |
| 4 | 17,250 | 74,234 | 1,434 | 80% | 20% |
| 5 | 17,250 | 75,559 | 1,425 | 77% | 23% |
| 6 | 17,250 | 74,588 | 1,432 | 82% | 18% |
| 7 | 17,250 | 73,035 | 1,379 | 65% | 35% |
| 8 | 17,250 | 70,156 | 1,403 | 78% | 22% |
| 9 | 17,250 | 72,907 | 1,384 | 67% | 33% |
| 10 | 17,250 | 69,852 | 1,407 | 73% | 27% |
| 11 | 17,250 | 70,278 | 1,396 | 100% | 0% |
| 12 | 17,250 | 71,343 | 1,353 | 48% | 52% |
| 13 | 17,250 | 71,516 | 1,343 | 48% | 52% |
| 14 | 17,250 | 71,439 | 1,332 | 40% | 60% |
| 15 | 17,250 | 81,719 | 1,230 | 19% | 81% |
| 16 | 40,258 | 242,565 | 1,120 |  |  |
|  |  |  |  | 66.30% | 33.70% |

## Initially Water Saturated Decane Flooded Artificially Fractured CT Numbers of Core#7 Right Matrix

Table D.23 CT Numbers of Slices for  Core#7 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7rm0 dry | area | dry mean | CT# | 7rm1 | area | 100 % water saturated mean | CT# | fraction |
| 1 | 40,258 | 213,168 | -10 | 1 | 40,258 | 214,062 | -10 |  |
| 2 | 17,250 | 125,002 | 1,163 | 2 | 17,250 | 77,536 | 1,431 | 0.2680 |
| 3 | 17,250 | 126,323 | 1,157 | 3 | 17,250 | 71,920 | 1,491 | 0.3335 |
| 4 | 17,250 | 127,310 | 1,153 | 4 | 17,250 | 72,644 | 1,483 | 0.3298 |
| 5 | 17,250 | 127,260 | 1,153 | 5 | 17,250 | 75,928 | 1,448 | 0.2945 |
| 6 | 17,250 | 123,889 | 1,167 | 6 | 17,250 | 73,648 | 1,472 | 0.3049 |
| 7 | 17,250 | 124,634 | 1,164 | 7 | 17,250 | 72,000 | 1,490 | 0.3256 |
| 8 | 17,250 | 125,829 | 1,159 | 8 | 17,250 | 68,813 | 1,524 | 0.3646 |
| 9 | 17,250 | 125,260 | 1,161 | 9 | 17,250 | 73,110 | 1,478 | 0.3164 |
| 10 | 17,250 | 127,267 | 1,153 | 10 | 17,250 | 70,339 | 1,507 | 0.3543 |
| 11 | 17,250 | 127,969 | 1,150 | 11 | 17,250 | 65,718 | 1,557 | 0.4065 |
| 12 | 17,250 | 126,160 | 1,158 | 12 | 17,250 | 71,540 | 1,495 | 0.3369 |
| 13 | 17,250 | 125,863 | 1,159 | 13 | 17,250 | 70,936 | 1,501 | 0.3421 |
| 14 | 17,250 | 126,737 | 1,155 | 14 | 17,250 | 73,835 | 1,470 | 0.3147 |
| 15 | 17,250 | 128,028 | 1,150 | 15 | 17,250 | 86,200 | 1,338 | 0.1880 |
| 16 | 40,258 | 218,805 | 0 | 16 | 40,258 | 218,494 | 0 |  |
|  |  |  |  |  |  |  | average  | 32.00% |

Table D.24 CT Numbers of Slices for  Core#7 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7rmw1 | area | 100 % water saturated mean | CT# | 7rmd1 | area | 100 % decane saturated mean | CT# |
| 1 | 40,258 | 214,062 | -10 | 1 | 40,258 | 172,707 | 630 |
| 2 | 17,250 | 77,536 | 1,431 | 2 | 40,258 | 102,303 | 1,185 |
| 3 | 17,250 | 71,920 | 1,491 | 3 | 40,258 | 96,680 | 1,215 |
| 4 | 17,250 | 72,644 | 1,483 | 4 | 40,258 | 98,655 | 1,204 |
| 5 | 17,250 | 75,928 | 1,448 | 5 | 40,258 | 99,409 | 1,200 |
| 6 | 17,250 | 73,648 | 1,472 | 6 | 40,258 | 99,947 | 1,197 |
| 7 | 17,250 | 72,000 | 1,490 | 7 | 40,258 | 99,439 | 1,200 |
| 8 | 17,250 | 68,813 | 1,524 | 8 | 40,258 | 99,354 | 1,201 |
| 9 | 17,250 | 73,110 | 1,478 | 9 | 40,258 | 96,924 | 1,214 |
| 10 | 17,250 | 70,339 | 1,507 | 10 | 40,258 | 97,069 | 1,213 |
| 11 | 17,250 | 65,718 | 1,557 | 11 | 40,258 | 97,784 | 1,209 |
| 12 | 17,250 | 71,540 | 1,495 | 12 | 40,258 | 98,440 | 1,205 |
| 13 | 17,250 | 70,936 | 1,501 | 13 | 40,258 | 101,023 | 1,192 |
| 14 | 17,250 | 73,835 | 1,470 | 14 | 40,258 | 98,52 | 1,205 |
| 15 | 17,250 | 86,200 | 1,338 | 15 | 40,258 | 99,039 | 1,202 |
| 16 | 40,258 | 218,494 | 1,100 | 16 | 40,258 | 173,334 | 631 |

Table D.25 CT Numbers of Slices for Core#7 Saturation Calculation.

| 7rmwd2 | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 190,462 | -11 |  |  |
| 2 | 17,250 | 66,338 | 1,296 | 45% | 55% |
| 3 | 17,250 | 60,774 | 1,279 | 23% | 77% |
| 4 | 17,250 | 61,950 | 1,309 | 38% | 62% |
| 5 | 17,250 | 62,594 | 1,293 | 37% | 63% |
| 6 | 17,250 | 62,098 | 1,305 | 39% | 61% |
| 7 | 17,250 | 65,680 | 1,323 | 42% | 58% |
| 8 | 17,250 | 64,031 | 1,357 | 48% | 52% |
| 9 | 17,250 | 65,303 | 1,325 | 42% | 58% |
| 10 | 17,250 | 63,775 | 1,361 | 50% | 50% |
| 11 | 17,250 | 64,540 | 1,356 | 42% | 58% |
| 12 | 17,250 | 67,379 | 1,343 | 48% | 52% |
| 13 | 17,250 | 68,054 | 1,341 | 48% | 52% |
| 14 | 17,250 | 68,789 | 1,342 | 52% | 48% |
| 15 | 17,250 | 75,669 | 1,220 | 13% | 87% |
| 16 | 40,258 | 216,568 | 50 |  |  |
|  |  |  |  | 40.63% | 59.37% |

Table D.26 CT Numbers of Slices for Core#7 Saturation Calculation.

| 7rmwd2 | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 218,127 | 650 |  |  |
| 2 | 17,250 | 78,505 | 1,237 | 100% | 0% |
| 3 | 17,250 | 76,131 | 1,258 | 47% | 53% |
| 4 | 17,250 | 78,422 | 1,238 | 55% | 45% |
| 5 | 17,250 | 79,778 | 1,226 | 76% | 24% |
| 6 | 17,250 | 78,727 | 1,235 | 61% | 39% |
| 7 | 17,250 | 76,150 | 1,258 | 45% | 55% |
| 8 | 17,250 | 74,432 | 1,273 | 41% | 59% |
| 9 | 17,250 | 76,373 | 1,256 | 71% | 29% |
| 10 | 17,250 | 74,261 | 1,274 | 45% | 55% |
| 11 | 17,250 | 73,903 | 1,278 | 55% | 45% |
| 12 | 17,250 | 74,865 | 1,269 | 58% | 42% |
| 13 | 17,250 | 74,070 | 1,276 | 44% | 56% |
| 14 | 17,250 | 74,576 | 1,272 | 51% | 49% |
| 15 | 17,250 | 82,712 | 1,200 | 100% | 0% |
| 16 | 40,258 | 239,973 | 450 |  |  |
|  |  |  |  | 60.92% | 39.08% |

Table D.27 CT Numbers of Slices for Core#7 Saturation Calculation after Gel Injection and Production.

| 7rmwdwg | area | water after decane and gel mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,258 | 200,922 | -100 |  |  |
| 2 | 17,250 | 74,876 | 1,374 | 77% | 23% |
| 3 | 17,250 | 76,315 | 1,457 | 88% | 12% |
| 4 | 17,250 | 73,834 | 1,439 | 84% | 16% |
| 5 | 17,250 | 75,159 | 1,430 | 93% | 7% |
| 6 | 17,250 | 74,188 | 1,437 | 87% | 13% |
| 7 | 17,250 | 72,635 | 1,384 | 63% | 37% |
| 8 | 17,250 | 69,756 | 1,408 | 64% | 36% |
| 9 | 17,250 | 72,507 | 1,389 | 67% | 33% |
| 10 | 17,250 | 69,452 | 1,412 | 68% | 32% |
| 11 | 17,250 | 69,878 | 1,401 | 100% | 0% |
| 12 | 17,250 | 70,943 | 1,358 | 53% | 47% |
| 13 | 17,250 | 71,116 | 1,348 | 51% | 49% |
| 14 | 17,250 | 71,039 | 1,337 | 50% | 50% |
| 15 | 17,250 | 81,319 | 1,235 | 24% | 76% |
| 16 | 40,258 | 242,565 | 1,120 |  |  |
|  |  |  |  | 69.19% | 30.81% |

## Initially Water Saturated Decane Flooded CT Scan Images of Core#5

Figure D.6 CT Slice Views of  Core#5  Dry.

Figure D.7 CT Slice Views of  Core#5 Water Saturated.

Figure D.8 CT Slice Views of Core#5 Before Production.

Figure D.9 CT Slice Views of  Core#5 After Production.

Figure D.10 CT Slice Views of  Core#5 After Polymer Gel Injection.

## Initially Water Saturated Decane Flooded CT Numbers of Core#5

Table D.28 CT Numbers of Slices for Core#5 Porosity Calculation.

| Porosity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51dry | area | dry mean | CT# | 52 | area | 100 % water saturated mean | CT# | Fraction |
| 1 | 40,260 | 239,878 | 50 | 1 | 40,260 | 200,023 | -5 |  |
| 2 | 40,260 | 144,736 | 1,006 | 2 | 40,260 | 78,932 | 1,357 | 0.3505 |
| 3 | 40,260 | 138,696 | 1,053 | 3 | 40,260 | 75,796 | 1,390 | 0.3372 |
| 4 | 40,260 | 139,651 | 1,045 | 4 | 40,260 | 75,594 | 1,392 | 0.3467 |
| 5 | 40,260 | 139,461 | 1,047 | 5 | 40,260 | 77,313 | 1,374 | 0.3270 |
| 6 | 40,260 | 137,574 | 1,061 | 6 | 40,260 | 78,642 | 1,360 | 0.2983 |
| 7 | 40,260 | 141,764 | 1,029 | 7 | 40,260 | 79,168 | 1,354 | 0.3250 |
| 8 | 40,260 | 134,769 | 1,083 | 8 | 40,260 | 82,793 | 1,316 | 0.2326 |
| 9 | 40,260 | 135,619 | 1,076 | 9 | 40,260 | 78,836 | 1,358 | 0.2811 |
| 10 | 40,260 | 136,543 | 1,069 | 10 | 40,260 | 75,317 | 1,395 | 0.3256 |
| 11 | 40,260 | 137,359 | 1,063 | 11 | 40,260 | 77,415 | 1,373 | 0.3097 |
| 12 | 40,260 | 141,832 | 1,029 | 12 | 40,260 | 80,118 | 1,344 | 0.3155 |
| 13 | 40,260 | 143,705 | 1,014 | 13 | 40,260 | 81,311 | 1,331 | 0.3173 |
| 14 | 40,260 | 140,242 | 1,041 | 14 | 40,260 | 86,073 | 1,281 | 0.2400 |
| 15 | 40,260 | 142,485 | 1,023 | 15 | 40,260 | 86,211 | 1,279 | 0.2558 |
| 16 | 40,260 | 170,984 | 360 | 16 | 40,260 | 169,692 | 1,100 |  |
|  |  |  |  |  |  |  | average  | 30.44% |

Table D.29 CT Numbers of Slices for  Core#5 Saturation Calculation.

| Saturation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5w1 | area | 100 % water saturated mean | CT# | 5d2 | area | 100 % decane saturated mean | CT# |
| 1 | 40,260 | 200,023 | -5 | 1 | 40,260 | 172,707 | 630 |
| 2 | 40,260 | 78,932 | 1,357 | 2 | 40,260 | 102,303 | 1,110 |
| 3 | 40,260 | 75,796 | 1,390 | 3 | 40,260 | 96,68 | 1,140 |
| 4 | 40,260 | 75,594 | 1,392 | 4 | 40,260 | 98,655 | 1,129 |
| 5 | 40,260 | 77,313 | 1,374 | 5 | 40,260 | 99,409 | 1,125 |
| 6 | 40,260 | 78,642 | 1,360 | 6 | 40,260 | 99,947 | 1,122 |
| 7 | 40,260 | 79,168 | 1,354 | 7 | 40,260 | 99,439 | 1,125 |
| 8 | 40,260 | 82,793 | 1,316 | 8 | 40,260 | 99,354 | 1,126 |
| 9 | 40,260 | 78,836 | 1,358 | 9 | 40,260 | 96,924 | 1,139 |
| 10 | 40,260 | 75,317 | 1,395 | 10 | 40,260 | 97,069 | 1,138 |
| 11 | 40,260 | 77,415 | 1,373 | 11 | 40,260 | 97,784 | 1,134 |
| 12 | 40,260 | 80,118 | 1,344 | 12 | 40,260 | 98,440 | 1,130 |
| 13 | 40,260 | 81,311 | 1,331 | 13 | 40,260 | 101,023 | 1,117 |
| 14 | 40,260 | 86,073 | 1,281 | 14 | 40,260 | 98,520 | 1,130 |
| 15 | 40,260 | 86,211 | 1,279 | 15 | 40,260 | 99,039 | 1,127 |
| 16 | 40,260 | 169,692 | 1,100 | 16 | 40,260 | 173,334 | 631 |

Table D.30 Saturation Calculation of CT Numbers of Slices for  Core#5 Before Production.

| 5wd2 | area | decane after water mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 233,190 | -5 |  |  |
| 2 | 40,260 | 80,131 | 1,233 | 50% | 50% |
| 3 | 40,260 | 77,839 | 1,261 | 48% | 52% |
| 4 | 40,260 | 78,449 | 1,254 | 47% | 53% |
| 5 | 40,260 | 79,984 | 1,235 | 44% | 56% |
| 6 | 40,260 | 83,852 | 1,188 | 27% | 73% |
| 7 | 40,260 | 82,994 | 1,198 | 32% | 68% |
| 8 | 40,260 | 84,748 | 1,177 | 27% | 73% |
| 9 | 40,260 | 79,229 | 1,244 | 48% | 52% |
| 10 | 40,260 | 77,022 | 1,271 | 52% | 48% |
| 11 | 40,260 | 79,848 | 1,237 | 43% | 57% |
| 12 | 40,260 | 80,709 | 1,226 | 45% | 55% |
| 13 | 40,260 | 79,984 | 1,235 | 55% | 45% |
| 14 | 40,260 | 83,852 | 1,188 | 38% | 62% |
| 15 | 40,260 | 82,994 | 1,198 | 47% | 53% |
| 16 | 40,260 | 170,112 | 350 |  |  |
|  |  |  |  | 43.12% | 56.88% |

Table D.31 Saturation Calculation of CT Numbers of Slices for Core#5 After Production.

| 5wdw | area | water after decane mean | CT# | saturation SW | saturation SD |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 228,446 | -5 |  |  |
| 2 | 40,260 | 76,940 | 1,334 | 91% | 9% |
| 3 | 40,260 | 71,325 | 1,426 | 100% | 0% |
| 4 | 40,260 | 73,766 | 1,386 | 98% | 2% |
| 5 | 40,260 | 76,293 | 1,345 | 88% | 12% |
| 6 | 40,260 | 80,518 | 1,276 | 65% | 35% |
| 7 | 40,260 | 80,478 | 1,277 | 66% | 34% |
| 8 | 40,260 | 81,673 | 1,257 | 69% | 31% |
| 9 | 40,260 | 78,260 | 1,313 | 80% | 20% |
| 10 | 40,260 | 76,478 | 1,347 | 81% | 19% |
| 11 | 40,260 | 80,938 | 1,269 | 57% | 43% |
| 12 | 40,260 | 80,646 | 1,274 | 67% | 33% |
| 13 | 40,260 | 80,083 | 1,283 | 78% | 22% |
| 14 | 40,260 | 86,608 | 1,177 | 31% | 69% |
| 15 | 40,260 | 87,771 | 1,158 | 20% | 80% |
| 16 | 40,260 | 170,300 | 350 |  |  |
|  |  |  |  | 70.75% | 29.25% |

Table D.32 Saturation Calculation of CT Numbers of Slices for Core#5 After Polymer Gel Injection.

| 5wdwg | area | water after decane and gel mean | CT# | saturation SW | saturation SW |
| --- | --- | --- | --- | --- | --- |
| 1 | 40,260 | 137,057 | 985 |  |  |
| 2 | 40,260 | 74,896 | 1,352 | 90% | 10% |
| 3 | 40,260 | 73,857 | 1,367 | 91% | 9% |
| 4 | 40,260 | 75,539 | 1,343 | 81% | 19% |
| 5 | 40,260 | 74,992 | 1,351 | 91% | 9% |
| 6 | 40,260 | 79,704 | 1,281 | 67% | 33% |
| 7 | 40,260 | 80,261 | 1,273 | 65% | 35% |
| 8 | 40,260 | 81,726 | 1,252 | 66% | 34% |
| 9 | 40,260 | 75,881 | 1,338 | 91% | 9% |
| 10 | 40,260 | 75,937 | 1,337 | 77% | 23% |
| 11 | 40,260 | 79,014 | 1,291 | 66% | 34% |
| 12 | 40,260 | 80,186 | 1,274 | 67% | 33% |
| 13 | 40,260 | 77,691 | 1,311 | 91% | 9% |
| 14 | 40,260 | 82,442 | 1,241 | 74% | 26% |
| 15 | 40,260 | 88,549 | 1,151 | 57% | 43% |
| 16 | 40,260 | 170,742 | 350 |  |  |
|  |  |  |  | 76.69% | 23.31% |